# Invitation for Bids

# CITY OF ABERDEEN

SIDEWALK/ HANDICAP RAMPS AND CURB REPAIRS ON MOUNT ROYAL AVENUE

BID NO.  13-02

State Contract No.: AX352B52

FAP No.: SRTS-1(913)E

Grant Year E

NOTE:  QUESTIONS CONCERNING THIS SOLICITATION

MUST BE FAXED OR E-MAILED TO:

Phyllis Grover

Department of Planning and Community Development

Email:	  pgrover@aberdeen-md.org

Phone:	410-272-1600 ext. 216

Fax:	 410-272-8163

NOTICE TO CONTRACTORS

When applicable, this Contract may have one or more of the following preset items in the Schedule of Prices: Price Adjustments (PA) for items such as Diesel Fuel, Asphalt Binder, Pavement Density, Hot Mix Asphalt Mixture and Surface Profile; Incentives/Disincentive (I/D) such as Quarterly and Final Erosion and Sediment Control Incentive/Liquidated Damages Payments, Milestone Completion I/D’s and Early Completion I/D’s; and Unanticipated Work (UW).  These prices were not used as a factor when determining the minority participation goal(s) (if applicable) for this contract.  Therefore Bidders should not include PA, I/D and UW funds when calculating minority participation.

If you should have any questions pertaining to this notice then contact the persons listed below:

City of Aberdeen 					Project Manager: Phyllis Grover

60 North Parke Street				pgrover@aberdeen-md.org

Aberdeen, MD 21001

410-272-1600 ext. 210

410-273-7402

A pre-bid meeting will be held on November 4, 2013 at 3:00 p.m. on Mount Royal Avenue and Williams Street (on site), Aberdeen MD 21001.

Written or faxed requests for clarification and additional information must be received in the Department of Planning and Community Development not later than November 8, 2013 to ensure adequate time to prepare and circulate any necessary addenda to all offerors.

One (1) original and one (1) sealed copies of the proposal must be received in the Department of Planning and Community Development, at the City of Aberdeen 60 North Parke Street, Aberdeen MD 21001  no later than 3:00 p.m. on  November 18, 2013.  There will not be a public opening. The proposal must be accompanied by a brief transmittal letter, signed by an officer authorized to bind the firm to its proposal, with required affidavit(s) attached.

TABLE OF CONTENTS

Invitation for Bids 										1

Notice to Contractors – Pre-Bidding Session							2

Table of Contents										3-4

CONTRACT PROVISIONS

CP - Form FHWA 1273										5

CP – Location and Signature Sheet								6

CP – Design Designation Chart									7

CP - National Cooperative Highway Research Program (NCHRP)

Report 350 Implementation Schedule							8-9

CP - Occupying Wetlands									10

CP - Federal-Aid Construction Contracts							11-14

CP - Affirmative Action Requirements								15

CP - Utilization of Disadvantaged Business Enterprises						16-24

CP - MBE/DBE Compliance Field Meeting							25

CP - Traffic Control Plan Certification								26

CP - Prevailing Wage Instructions for the Contractor						27-28

CP – Davis Bacon Wage Determination 		(pages 1 of 4)					29-32

CP - Notice of Actions for Affirmative Action

Ensure Equal Employment Opportunity (Executive Order 11246)				33-40

CP - Training Provisions									41-43

CP - High Visibility Safety Apparel Policy							44-45					          SPECIAL PROVISIONS

SP - Project Description, Specifications, Notice to contractors, and Employment Agency		46-49

GENERAL PROVISIONS

# Invitation to Bid, Issuing Office, Request for Proposal, Schedule, and Reservations		50-51

Addenda, Wage Rates, Economy of Preparation, Incurred Expenses,

Acceptance of Terms and Conditions, Public Information Act Notice,

Evidence of Offeror Responsibility, Award without Discussions					52

Contractor Responsibilities; Subcontractors, Conflicts of Interest, Financial Disclosure,

Political Contribution Disclosure, Anti-Bribery Affidavit, Non-collusion				52-53

Compliance with Laws, Governing Law,

Ownership and Retention of Records, Billing and Payment

Applicable Standards/Guidelines, City Project Manager, Insurance Requirements			54

Evaluation and Selection Criteria, Description of the Project, Daily Cleanup,

Performance Surety and Maintenance Surety							55

special Provisions

Material Section 902-Portland Cement Concrete and Related Products		 56-70

Section 916-Soil and Soil-Aggregate Borrow					       71

Section 925-Detectable Warning Surfaces                                                                       72

Physical Properties                                                                                                            73

Proposal Form Packet — Federal							        74-100

Contract Agreement for the City of Aberdeen					        101-103

Attachment A

Bid Form									        104-105

Concrete Mountable Curb and Gutter Plate S-2					        106

7” Combination Curb and Gutter S-1						        107

Sidewalk Ramp Detail Perpendicular S-7					        108

Sidewalk Ramps Parallel S-8							        109

Sidewalk Ramps Parallel S-9							        110

Detectable Warning Surface S-10						        111

Standard 5" Sidewalk S-11							        112

Curb and Gutter Transition at Inlets S-36					        113

Standard No. MD 615.01	              					        114

NOTICE TO CONTRACTORS

FHWA Form-1273

This NTC goes in all  projects

Contractors will comply with the current language of the FHWA-1273 specification.

Environmental Stewardship

The Maryland State Highway Administration is committed to the development and maintenance of the Administration’s highway system in an environmentally responsible manner.  Therefore, Contractors are encouraged to consider the use of Administration-approved recycled and reclaimed materials in construction projects where practicable, and in accordance with the Plans and Specifications.

The Contractor is also encouraged to reuse, salvage, or recycle all generated waste materials to the extent possible.  Materials that are easily recognizable, maintain their physical properties, meet the required material properties for recycling, are easily separated and transported, and have value as commodities are candidates for recycling.  These types of materials generally include metals (steel, iron, copper, aluminum, bronze, etc.), plastics (cones, barrels, barricades, crash cushion plastic barrels, conduit, containers, etc.), aluminum poles and signs, electronic and electrical components, signals and signal components, topsoil, formwork, temporary false work, brick, masonry, stone, wood, paper, and timber and yard waste from clearing and grubbing operations.

S.H.A. CONTRACT NO. : AX352B52

FEDERAL AID PROJECT NO: SRTS-1(913)E

SCALE: NONE	LOCATION MAP		CITY OF ABERDEEN

"I/We hereby certify that any clearing, grading construction and/or development will be done pursuant to this plan and that any responsible personnel involved in the construction project will have a certificate of attendance at a Maryland Department of the Environment approved training program for the control of erosion and sediment before beginning the project."

"I hereby authorize the right of entry for periodic on-site evaluation by State of Maryland, Department of the Environment, compliance inspectors."

REVIEWED AND

APPROVAL RECOMMENDED

PROJECT MANAGER

APPROVAL RECOMMENDED

DIRECTOR OF PUBLIC WORKS

APPROVED

CITY ENGINEER

DESIGN DESIGNATION CHART

| DESIGN DESIGNATION |  |  |
| --- | --- | --- |
| Control\Years |  |  |
| Average Daily Traffic (A.D.T.) |  |  |
| Design Hourly Volume (D.H.V.) |  |  |
| Directional Distribution |  |  |
| % Trucks – A.D.T. |  |  |
| % Trucks – D.H.V. |  |  |
| Design Speed | 25 mph |  |
| Functional Classifications |  |  |
| Controls of Access |  |  |
| Intensity of Development | Urban street |  |
| Terrain | Flat |  |
| Anticipated Posted Speed | 25 mph |  |

7

NOTICE TO ALL HOLDERS OF THIS CONTRACT DOCUMENT

NATIONAL COOPERATIVE HIGHWAY RESEARCH PROGRAM (NCHRP) REPORT 350 IMPLEMENTATION SCHEDULE FOR DEVICES USED IN THE MAINTENANCE OF TRAFFIC

Except as otherwise specified in this Section, all items for the maintenance of traffic, including those listed under the following categories, shall be crashworthy in conformance with Level 3 or other Level as specified by the Engineer in conformance with the safety crash testing and performance criteria published in the National Cooperative Highway Research Program (NCHRP) Report 350, “Recommended Procedures for the Safety Performance Evaluation of Highway Features.”  When conformance with NCHRP Report 350 is required, the Contractor shall provide the Engineer with the manufacturers’ certifications that the devices comply with the specified criteria.

Unless specifically waived by an attachment to these Contract Provisions, devices must be approved by the Office of Traffic and Safety.

Category 1 Devices

These devices are cones, tubular markers, flexible delineator posts, and drums, all without any accessories or attachments, which are used for channelization and delineation.

Category 2 Devices

These devices are Type I, II, and III barricades; portable sign supports with signs; intrusion alarms; and drums, vertical panels, and cones, all with accessories or attachments.

Category 3 Devices

(a)	Truck Mounted Attenuators (TMAs).

(b)	Temporary Barrier.

(1)	Concrete Barrier.

(2)	Traffic Barrier W Beam and Water Filled Barrier.

(c)	Temporary End Treatments.

Category 4 Devices

These devices are area lighting supports, arrow panels, and portable variable message signs that are usually portable or trailer-mounted.

8

| WORK ZONE DEVICES | IMPLEMENTATION SCHEDULE TO CONFORM TO NCHRP REPORT 350 CRITERIA |
| --- | --- |
| CATEGORY 1 Cones, tubular markers, flexible delineator posts, and drums (all without any accessories or attachments) | All devices shall conform to NCHRP Report 350 criteria. |
| CATEGORY 2 Type I, II, and III barricades; portable signs supports with signs; intrusion alarms; and drums, vertical panels, and cones (all with accessories or attachments) | All devices shall conform to NCHRP Report 350 criteria. |
| CATEGORY 3 (a) Truck Mounted Attenuators (TMA) (b) Temporary Barriers (1) Concrete Barrier (2) Traffic Barrier W Beam and Water Filled Barrier (c) Temporary End Treatments | All devices shall conform to NCHRP Report 350 criteria. |
| CATEGORY 4 Portable trailer mounted devices including area lighting supports, arrow panels, and changeable message signs | The Contractor may use devices that do not conform to NCHRP Report 350 criteria, until compliance dates are established. Use of these devices shall comply with the provisions of Part 6 of the MUTCD. |

9

OCCUPYING WETLANDS

The Contractor is hereby alerted to the importance of preserving wetland areas.  The Administration, in conjunction with the various environmental agencies, has developed these Contract Documents so as to minimize or eliminate disturbance and damage to existing wetland areas.  In order to accomplish this, the following must be rigidly adhered to:

(a)	Prior to performing any work on the project, the areas of wetland will be identified and marked as directed by the Administration.  All personnel of the Contractor or sub-contractors shall be alerted to these designated areas.

(b)	The Contractor or sub-contractors shall not impact any wetland or waterway, whether it is permanently or temporarily unless otherwise stipulated in the permit application and approved as an authorized action by the appropriate regulatory agency.  No fill shall be placed in these areas without a permit.

(c)	If a Contractor or sub-contractor has to impact a wetland or waterway that is not covered by an existing wetland permit, they shall immediately notify the Engineer.  The Engineer will notify the Environmental Programs Division to determine the extent of any permit modification.  At that time the Environmental Programs Division will request a permit modification or submit a permit application.

(d)	If the Contractor impacts any wetland or waterway for which they do not have a wetland permit, they shall be responsible for restoring the wetland areas and possibly mitigating the wetland impacts to the full satisfaction of the environmental agencies, which could include monetary compensation.

(e)	The cost of restoration and mitigation of the impacted areas shall be at no additional cost to the Administration.

The importance of not abusing the wetland areas cannot be overemphasized.  Abuse of wetland areas could jeopardize the operation of the total Contract and could be cause for a shut-down.  If a shut-down occurs because of the Contractor's failure to secure the required permits (i.e. the Contractor’s method of work includes impacts not approved by previously acquired permits), the Contractor’s negligence or operations, all costs and damages to the Contractor and to the State will be at no additional cost to the Administration.  Noncompliance with these requirements will not be considered for an extension of Contract time.

10

I.	General	   1

II.	Nondiscrimination	   1

III.	Nonsegregated Facilities	   3

IV.	Payment of Predetermined Minimum Wage	   3

V.	Statements and Payrolls	   6

VI.	Record of Materials, Supplies, and Labor	   6

VII.	Subletting or Assigning the Contract	   7

VIII.	Safety:  Accident Prevention	   7

IX.	False Statements Concerning Highway Projects	   7

X.	Implementation of Clean Air Act and Federal

Water Pollution Control Act	   8

XI.	Certification Regarding Debarment, Suspension,

Ineligibility, and Voluntary Exclusion	   8

XII.	Certification Regarding Use of Contract Funds for

Lobbying	  9

ATTACHMENTS

A.	Employment Preference for Appalachian Contracts

(included in Appalachian contracts only)

I.  GENERAL

1.  These contract provisions shall apply to all work performed on the contract by the contractor's own organization and with the assistance of workers under the contractor's immediate superintendence and to all work performed on the contract by piecework, station work, or by subcontract.

2.  Except as otherwise provided for in each section, the contractor shall insert in each subcontract all of the stipulations contained in these Required Contract Provisions, and further require their inclusion in any lower tier subcontract or purchase order that may in turn be made.  The Required Contract Provisions shall not be incorporated by reference in any case.  The prime contractor shall be responsible for compliance by any subcontractor or lower tier subcontractor with these Required Contract Provisions.

3.  A breach of any of the stipulations contained in these Required Contract Provisions shall be sufficient grounds for termination of the contract.

4.  A breach of the following clauses of the Required Contract Provisions may also be grounds for debarment as provided in 29 CFR 5.12:

Section I, paragraph 2;

Section IV, paragraphs 1, 2, 3, 4, and 7;

Section V, paragraphs 1 and 2a through 2g.

5.  Disputes arising out of the labor standards provisions of Section IV (except paragraph 5) and Section V of these Required Contract Provisions shall not be subject to the general disputes clause of this contract.  Such disputes shall be resolved in accordance with the procedures of the U.S. Department of Labor (DOL) as set forth in 29 CFR 5, 6, and 7.  Disputes within the meaning of this clause include disputes between the contractor (or any of its subcontractors) and the contracting agency, the DOL, or the contractor's employees or their representatives.

6.  Selection of Labor:  During the performance of this contract, the contractor shall not:

A. discriminate against labor from any other State, possession, or territory of the United States (except for employment preference for Appalachian contracts, when applicable, as specified in Attachment A), or

B. employ convict labor for any purpose within the limits of the project unless it is labor performed by convicts who are on parole, supervised release, or probation.

II.   NONDISCRIMINATION

(Applicable to all Federal-aid construction contracts and to all related subcontracts of $10,000 or more.)

1.  Equal Employment Opportunity:  Equal employment opportunity (EEO) requirements not to discriminate and to take affirmative action to assure equal opportunity as set forth under laws, executive orders, rules, regulations (28 CFR 35, 29 CFR 1630 and 41 CFR 60) and orders of the Secretary of Labor as modified by the provisions  prescribed herein, and imposed pursuant to 23 U.S.C. 140 shall constitute the EEO and specific affirmative action standards for the contractor's project activities under this contract.  The Equal Opportunity Construction Contract Specifications set forth under 41 CFR 60-4.3 and the provisions of the American Disabilities Act of 1990 (42 U.S.C. 12101 et seq.) set forth under 28 CFR 35 and 29 CFR 1630 are incorporated by reference in this contract.  In the execution of this contract, the contractor agrees to comply with the following minimum specific requirement activities of EEO:

a. The contractor will work with the State highway agency (SHA) and the Federal Government in carrying out EEO obligations and in their review of his/her activities under the contract.

b. The contractor will accept as his operating policy the following statement:

"It is the policy of this Company to assure that applicants are employed, and that employees are treated during employment, without regard to their race, religion, sex, color, national origin, age or disability.  Such action shall include: employment, upgrading, demotion, or transfer; recruitment or recruitment advertising; layoff or termination; rates of pay or other forms of compensation; and selection for training, including apprenticeship, preapprenticeship, and/or on-the-job training."

2.  EEO Officer:  The contractor will designate and make known to the SHA contracting officers an EEO Officer who will have the responsibility for and must be capable of effectively administering and promoting an active contractor program of EEO and who must be assigned adequate authority and responsibility to do so.

3.  Dissemination of Policy:  All members of the contractor's staff who are authorized to hire, supervise, promote, and discharge employees, or who recommend such action, or who are substantially involved in such action, will be made fully cognizant of, and will implement, the contractor's EEO policy and contractual responsibilities to provide EEO in each grade and classification of employment.  To ensure that the above agreement will be met, the following actions will be taken as a minimum:

a.  Periodic meetings of supervisory and personnel office employees will be conducted before the start of work and then not less often than once every six months, at which time the contractor's EEO policy and its implementation will be reviewed and explained.  The meetings will be conducted by the EEO Officer.

b. All new supervisory or personnel office employees will be given a thorough indoctrination by the EEO Officer, covering all major aspects of the contractor's EEO obligations within thirty days following their reporting for duty with the contractor.

c. All personnel who are engaged in direct recruitment for the project will be instructed by the EEO Officer in the contractor's procedures for locating and hiring minority group employees.

d. Notices and posters setting forth the contractor's EEO policy will be placed in areas readily accessible to employees, applicants for employment and potential employees.

e. The contractor's EEO policy and the procedures to implement such policy will be brought to the attention of employees by means of meetings, employee handbooks, or other appropriate means.

4.  Recruitment:  When advertising for employees, the contractor will include in all advertisements for employees the notation:  "An Equal Opportunity Employer."  All such advertisements will be placed in publications having a large circulation among minority groups in the area from which the project work force would normally be derived.

a. The contractor will, unless precluded by a valid bargaining agreement, conduct systematic and direct recruitment through public and private employee referral sources likely to yield qualified minority group applicants.  To meet this requirement, the contractor will identify sources of potential minority group employees, and establish with such identified sources procedures whereby minority group applicants may be referred to the contractor for employment consideration.

b. In the event the contractor has a valid bargaining agreement providing for exclusive hiring hall referrals, he is expected to observe the provisions of that agreement to the extent that the system permits the contractor's compliance with EEO contract provisions.  (The DOL has held that where implementation of such agreements have the effect of discriminating against minorities or women, or obligates the contractor to do the same, such implementation violates Executive Order 11246, as amended.)

c. The contractor will encourage his present employees to refer minority group applicants for employment.  Information and procedures with regard to referring minority group applicants will be discussed with employees.

5.  Personnel Actions:  Wages, working conditions, and employee benefits shall be established and administered, and personnel actions of every type, including hiring, upgrading, promotion, transfer, demotion, layoff, and termination, shall be taken without regard to race, color, religion, sex, national origin, age or disability.  The following procedures shall be followed:

a. The contractor will conduct periodic inspections of project sites to insure that working conditions and employee facilities do not indicate discriminatory treatment of project site personnel.

b. The contractor will periodically evaluate the spread of wages paid within each classification to determine any evidence of discriminatory wage practices.

c. The contractor will periodically review selected personnel actions in depth to determine whether there is evidence of discrimination.  Where evidence is found, the contractor will promptly take corrective action.  If the review indicates that the discrimination may extend beyond the actions reviewed, such corrective action shall include all affected persons.

d. The contractor will promptly investigate all complaints of alleged discrimination made to the contractor in connection with his obligations under this contract, will attempt to resolve such complaints, and will take appropriate corrective action within a reasonable time.  If the investigation indicates that the discrimination may affect persons other than the complainant, such corrective action shall include such other persons.  Upon completion of each investigation, the contractor will inform every complainant of all of his avenues of appeal.

6.  Training and Promotion:

a. The contractor will assist in locating, qualifying, and increasing the skills of minority group and women employees, and applicants for employment.

b.  Consistent with the contractor's work force requirements and as permissible under Federal and State regulations, the contractor shall make full use of training programs, i.e., apprenticeship, and on-the-job training programs for the geographical area of contract performance.  Where feasible, 25 percent of apprentices or trainees in each occupation shall be in their first year of apprenticeship or training.  In the event a special provision for training is provided under this contract, this subparagraph will be superseded as indicated in the special provision.

c. The contractor will advise employees and applicants for employment of available training programs and entrance requirements for each.

d. The contractor will periodically review the training and promotion potential of minority group and women employees and will encourage eligible employees to apply for such training and promotion.

7.  Unions:  If the contractor relies in whole or in part upon unions as a source of employees, the contractor will use his/her best efforts to obtain the cooperation of such unions to increase opportunities for minority groups and women within the unions, and to effect referrals by such unions of minority and female employees.  Actions by the contractor either directly or through a contractor's association acting as agent will include the procedures set forth below:

a.  The contractor will use best efforts to develop, in cooperation with the unions, joint training programs aimed toward qualifying more minority group members and women for membership in the unions and increasing the skills of minority group employees and women so that they may qualify for higher paying employment.

b. The contractor will use best efforts to incorporate an EEO clause into each union agreement to the end that such union will be contractually bound to refer applicants without regard to their race, color, religion, sex, national origin, age or disability.

c. The contractor is to obtain information as to the referral practices and policies of the labor union except that to the extent such information is within the exclusive possession of the labor union and such labor union refuses to furnish such information to the contractor, the contractor shall so certify to the SHA and shall set forth what efforts have been made to obtain such information.

d. In the event the union is unable to provide the contractor with a reasonable flow of minority and women referrals within the time limit set forth in the collective bargaining agreement, the contractor will, through independent recruitment efforts, fill the employment vacancies without regard to race, color, religion, sex, national origin, age or disability; making full efforts to obtain qualified and/or qualifiable minority group persons and women.  (The DOL has held that it shall be no excuse that the union with which the contractor has a collective bargaining agreement providing for exclusive referral failed to refer minority employees.)  In the event the union referral practice prevents the contractor from meeting the obligations pursuant to Executive Order 11246, as amended, and these special provisions, such contractor shall immediately notify the SHA.

8.  Selection of Subcontractors, Procurement of Materials and Leasing of Equipment:  The contractor shall not discriminate on the grounds of race, color, religion, sex, national origin, age or disability in the selection and retention of subcontractors, including procurement of materials and leases of equipment.

a. The contractor shall notify all potential subcontractors and suppliers of his/her EEO obligations under this contract.

b.  Disadvantaged business enterprises (DBE), as defined in 49 CFR 23, shall have equal opportunity to compete for and perform subcontracts which the contractor enters into pursuant to this contract.  The contractor will use his best efforts to solicit bids from and to utilize DBE subcontractors or subcontractors with meaningful minority group and female representation among their employees.  Contractors shall obtain lists of DBE construction firms from SHA personnel.

11

c. The contractor will use his best efforts to ensure subcontractor compliance with their EEO obligations.

9.  Records and Reports:  The contractor shall keep such records as necessary to document compliance with the EEO requirements.  Such records shall be retained for a period of three years following completion of the contract work and shall be available at reasonable times and places for inspection by authorized representatives of the SHA and the FHWA.

a. The records kept by the contractor shall document the following:

(1)  The number of minority and non-minority group members and women employed in each work classification on the project;

(2)  The progress and efforts being made in cooperation with unions, when applicable, to increase employment opportunities for minorities and women;

(3)  The progress and efforts being made in locating, hiring, training, qualifying, and upgrading minority and female employees; and

(4)  The progress and efforts being made in securing the services of DBE subcontractors or subcontractors with meaningful minority and female representation among their employees.

b.  The contractors will submit an annual report to the SHA each July for the duration of the project, indicating the number of minority, women, and non-minority group employees currently engaged in each work classification required by the contract work.  This information is to be reported on Form FHWA-1391.  If on-thejob training is being required by special provision, the contractor will be required to collect and report training data.

III.  NONSEGREGATED FACILITIES

(Applicable to all Federal-aid construction contracts and to all related subcontracts of $10,000 or more.)

a.  By submission of this bid, the execution of this contract or subcontract, or the consummation of this material supply agreement or purchase order, as appropriate, the bidder, Federal-aid construction contractor, subcontractor, material supplier, or vendor, as appropriate, certifies that the firm does not maintain or provide for its employees any segregated facilities at any of its establishments, and that the firm does not permit its employees to perform their services at any location, under its control, where segregated facilities are maintained.  The firm agrees that a breach of this certification is a violation of the EEO provisions of this contract.  The firm further certifies that no employee will be denied access to adequate facilities on the basis of sex or disability.

b.  As used in this certification, the term "segregated facilities" means any waiting rooms, work areas, restrooms and washrooms, restaurants and other eating areas, timeclocks, locker rooms, and other storage or dressing areas, parking lots, drinking fountains, recreation or entertainment areas, transportation, and housing facilities provided for employees which are segregated by explicit directive, or are, in fact, segregated on the basis of race, color, religion, national origin, age or disability, because of habit, local custom, or otherwise.  The only exception will be for the disabled when the demands for accessibility override (e.g. disabled parking).

c.  The contractor agrees that it has obtained or will obtain identical certification from proposed subcontractors or material suppliers prior to award of subcontracts or consummation of material supply agreements of $10,000 or more and that it will retain such certifications in its files.

IV. PAYMENT OF PREDETERMINED MINIMUM WAGE

(Applicable to all Federal-aid construction contracts exceeding $2,000 and to all related subcontracts, except for projects located on roadways classified as local roads or rural minor collectors, which are exempt.)

1.  General:

a. All mechanics and laborers employed or working upon the site of the work will be paid unconditionally and not less often than once a week and without subsequent deduction or rebate on any account [except such payroll deductions as are permitted by regulations (29 CFR 3) issued by the Secretary of Labor under the Copeland Act (40 U.S.C. 276c)] the full amounts of wages and bona fide fringe benefits (or cash equivalents thereof) due at time of payment.  The payment shall be computed at wage rates not less than those contained in the wage determination of the Secretary of Labor (hereinafter "the wage determination") which is attached hereto and made a part hereof, regardless of any contractual relationship which may be alleged to exist between the contractor or its subcontractors and such laborers and mechanics.  The wage determination (including any additional classifications and wage rates conformed under paragraph 2 of this Section IV and the DOL poster (WH-1321) or Form FHWA-1495) shall be posted at all times by the contractor and its subcontractors at the site of the work in a prominent and accessible place where it can be easily seen by the workers.  For the purpose of this Section, contributions made or costs reasonably anticipated for bona fide fringe benefits under Section 1(b)(2) of the Davis-Bacon Act (40 U.S.C. 276a) on behalf of laborers or mechanics are considered wages paid to such laborers or mechanics, subject to the provisions of Section IV, paragraph 3b, hereof.  Also, for the purpose of this Section, regular contributions made or costs incurred for more than a weekly period (but not less often than quarterly) under plans, funds, or programs, which cover the particular weekly period, are deemed to be constructively made or incurred during such weekly period.  Such laborers and mechanics shall be paid the appropriate wage rate and fringe benefits on the wage determination for the classification of work actually performed, without regard to skill, except as provided in paragraphs 4 and 5 of this Section IV.

b.  Laborers or mechanics performing work in more than one classification may be compensated at the rate specified for each classification for the time actually worked therein, provided, that the employer's payroll records accurately set forth the time spent in each classification in which work is performed.

c. All rulings and interpretations of the Davis-Bacon Act and related acts contained in 29 CFR 1, 3, and 5 are herein incorporated by reference in this contract.

2.  Classification:

a. The SHA contracting officer shall require that any class of laborers or mechanics employed under the contract, which is not listed in the wage determination, shall be classified in conformance with the wage determination.

b. The contracting officer shall approve an additional classification, wage rate and fringe benefits only when the following criteria have been met:

(1)  the work to be performed by the additional classification requested is not performed by a classification in the wage determination;

(2)  the additional classification is utilized in the area by the construction industry;

(3)  the proposed wage rate, including any bona fide fringe benefits, bears a reasonable relationship to the wage rates contained in the wage determination; and

(4)  with respect to helpers as defined in Section IV.4(c), when such a classification prevails in the area in which the work is performed.

c.  If the contractor or subcontractors, as appropriate, the laborers and mechanics (if known) to be employed in the additional classification or their representatives, and the contracting officer agree on the classification and wage rate (including the amount designated for fringe benefits where appropriate), a report of the action taken shall be sent by the contracting officer to the DOL, Administrator of the Wage and Hour Division, Employment Standards Administration, Washington, D.C. 20210.  The Wage and Hour Administrator, or an authorized representative, will approve, modify, or disapprove every additional classification action within 30 days of receipt and so advise the contracting officer or will notify the contracting officer within the 30-day period that additional time is necessary.

d.  In the event the contractor or subcontractors, as appropriate, the laborers or mechanics to be employed in the additional classification or their representatives, and the contracting officer do not agree on the proposed classification and wage rate (including the amount designated for fringe benefits, where appropriate), the contracting officer shall refer the questions, including the views of all interested parties and the recommendation of the contracting officer, to the Wage and Hour Administrator for determination.  Said Administrator, or an authorized representative, will issue a determination within 30 days of receipt and so advise the contracting officer or will notify the contracting officer within the 30-day period that additional time is necessary

e.  The wage rate (including fringe benefits where appropriate) determined pursuant to paragraph 2c or 2d of this Section IV shall be paid to all workers performing work in the additional classification from the first day on which work is performed in the classification.

3.  Payment of Fringe Benefits:

a.  Whenever the minimum wage rate prescribed in the contract for a  class of laborers or mechanics includes a fringe benefit which is not expressed as an hourly rate, the contractor or subcontractors, as appropriate, shall either pay the benefit as stated in the wage determination or shall pay another bona fide fringe benefit or an hourly case equivalent thereof.

b.  If the contractor or subcontractor, as appropriate, does not make payments to a trustee or other third person, he/she may consider as a part of the wages of any laborer or mechanic the amount of any costs reasonably anticipated in providing bona fide fringe benefits under a plan or program, provided, that the Secretary of Labor has found, upon the written request of the contractor, that the applicable standards of the Davis-Bacon Act have been met.  The Secretary of Labor may require the contractor to set aside in a separate account assets for the meeting of obligations under the plan or program.

4.	Apprentices and Trainees (Programs of the U.S. DOL) and Helpers:

a. Apprentices:

(1)  Apprentices will be permitted to work at less than the predetermined rate for the work they performed when they are employed pursuant to and individually registered in a bona fide apprenticeship program registered with the DOL, Employment and Training Administration, Bureau of Apprenticeship and Training, or with a State apprenticeship agency recognized by the Bureau, or if a person is  employed in his/her first 90 days of probationary employment as an apprentice in such an apprenticeship program, who is not individually registered in the program, but who has been certified by the Bureau of Apprenticeship and Training or a State apprenticeship agency (where appropriate) to be eligible for probationary employment as an apprentice.

(2)  The allowable ratio of apprentices to journeyman-level employees on the job site in any craft classification shall not be greater than the ratio permitted to the contractor as to the entire work force under the registered program.  Any employee listed on a payroll at an apprentice wage rate, who is not registered or otherwise employed as stated above, shall be paid not less than the applicable wage rate listed in the wage determination for the classification of work actually performed.  In addition, any apprentice performing work on the job site in excess of the ratio permitted under the registered program shall be paid not less than the applicable wage rate on the wage determination for the work

actually performed.  Where a contractor or subcontractor is performing construction on a project in a locality other than that in which its program is registered, the ratios and wage rates (expressed in percentages of the journeyman-level hourly rate) specified in the contractor's or subcontractor's registered program shall be observed.

(3)  Every apprentice must be paid at not less than the rate specified in the registered program for the apprentice's level of progress, expressed as a percentage of the journeyman-level hourly rate specified in the applicable wage determination.  Apprentices shall be paid fringe benefits in accordance with the provisions of the apprenticeship program.  If the apprenticeship program does not specify fringe benefits, apprentices must be paid the full amount of fringe benefits listed on the wage determination for the applicable classification.  If the Administrator for the Wage and Hour Division determines that a different practice prevails for the applicable apprentice classification, fringes shall be paid in accordance with that determination.

(4)  In the event the Bureau of Apprenticeship and Training, or a State apprenticeship agency recognized by the Bureau, withdraws approval of an apprenticeship program, the contractor or subcontractor will no longer be permitted to utilize apprentices at less than the applicable predetermined rate for the comparable work performed by regular employees until an acceptable program is approved.

b. Trainees:

(1)  Except as provided in 29 CFR 5.16, trainees will not be permitted to work at less than the predetermined rate for the work performed unless they are employed pursuant to and individually registered in a program which has received prior approval, evidenced by formal certification by the DOL, Employment and Training Administration.

(2)  The ratio of trainees to journeyman-level employees on the job site shall not be greater than permitted under the plan approved by the Employment and Training Administration.  Any employee listed on the payroll at a trainee rate who is not registered and participating in a training plan approved by the Employment and Training Administration shall be paid not less than the applicable wage rate on the wage determination for the classification of work actually performed.  In addition, any trainee performing work on the job site in excess of the ratio permitted under the registered program shall be paid not less than the applicable wage rate on the wage determination for the work actually performed.

(3)  Every trainee must be paid at not less than the rate specified in the approved program for his/her level of progress, expressed as a percentage of the journeyman-level hourly rate specified in the applicable wage determination.  Trainees shall be paid fringe benefits in accordance with the provisions of the trainee program.  If the trainee program does not mention fringe benefits, trainees shall be paid the full amount of fringe benefits listed on the wage determination unless the Administrator of the Wage and Hour Division determines that there is an apprenticeship program associated with the corresponding journeyman-level wage rate on the wage determination which provides for less than full fringe benefits for apprentices, in which case such trainees shall receive the same fringe benefits as apprentices.

(4)  In the event the Employment and Training Administration withdraws approval of a training program, the contractor or subcontractor will no longer be permitted to utilize trainees at less than the applicable predetermined rate for the work performed until an acceptable program is approved.

c.  Helpers:

Helpers will be permitted to work on a project if the helper classification is specified on an applicable wage determination or is approved pursuant to the conformance procedure set forth in Section IV.2.  Any worker listed on a payroll at a helper wage rate, shall be paid not less than the applicable wage rate on the wage determination for the classification of work actually performed.

5.  Apprentices and Trainees (Programs of the U.S. DOT):

12

Apprentices and trainees working under apprenticeship and skill training programs which have been certified by the Secretary of Transportation as promoting EEO in connection with Federal-aid highway construction programs are not subject to the requirements of paragraph 4 of this Section IV.  The straight time hourly wage rates for apprentices and trainees under such programs will be established by the particular programs.  The ratio of apprentices and trainees to journeymen shall not be greater than permitted by the terms of the particular program.

6.  Withholding:

The SHA shall upon its own action or upon written request of an authorized representative of the DOL withhold, or cause to be withheld, from the contractor or subcontractor under this contract or any other Federal contract with the same prime contractor, or any other Federally-assisted contract subject to Davis-Bacon prevailing wage requirements which is held by the same prime contractor, as much of the accrued payments or advances as may be considered necessary to pay laborers and mechanics, including apprentices, trainees, and helpers, employed by the contractor or any subcontractor the full amount of wages required by the contract.  In the event of failure to pay any laborer or mechanic, including any apprentice, trainee, or helper, employed or working on the site of the work, all or part of the wages required by the contract, the SHA contracting officer may, after written notice to the contractor, take such action as may be necessary to cause the suspension of any further payment, advance, or guarantee of funds until such violations have ceased.

7.  Overtime Requirements:

No contractor or subcontractor contracting for any part of the contract work which may require or involve the employment of laborers, mechanics, watchmen, or guards (including apprentices, trainees, and helpers described in paragraphs 4 and 5 above) shall require or permit any laborer, mechanic, watchman, or guard in any workweek in which he/she is employed on such work, to work in excess of 40 hours in such workweek unless such laborer, mechanic, watchman, or guard receives compensation at a rate not less than one-and-one-half times his/her basic rate of pay for all hours worked in excess of 40 hours in such workweek.

8.  Violation:

Liability for Unpaid Wages; Liquidated Damages:  In the event of any violation of the clause set forth in paragraph 7 above, the contractor and any subcontractor responsible thereof shall be liable to the affected employee for his/her unpaid wages.  In addition, such contractor and subcontractor shall be liable to the United States (in the case of work done under contract for the District of Columbia or a territory, to such District or to such territory) for liquidated damages.  Such liquidated damages shall be computed with respect to each individual laborer, mechanic, watchman, or guard employed in violation of the clause set forth in paragraph 7, in the sum of $10 for each calendar day on which

such employee was required or permitted to work in excess of the standard work week of 40 hours without payment of the overtime wages required by the clause set forth in paragraph 7.

9.  Withholding for Unpaid Wages and Liquidated Damages:

The SHA shall upon its own action or upon written request of any authorized representative of the DOL withhold, or cause to be withheld, from any monies payable on account of work performed by the contractor or subcontractor under any such contract or any other Federal contract with the same prime contractor, or any other Federally-assisted contract subject to the Contract Work Hours and Safety Standards Act, which is held by the same prime contractor, such sums as may be determined to be necessary to satisfy any liabilities of such contractor or subcontractor for unpaid wages and liquidated damages as provided in the clause set forth in paragraph 8 above.

V.  STATEMENTS AND PAYROLLS

(Applicable to all Federal-aid construction contracts exceeding $2,000 and to all related subcontracts, except for projects located on roadways classified as local roads or rural collectors, which are exempt.)

1.  Compliance with Copeland Regulations (29 CFR 3):

The contractor shall comply with the Copeland Regulations of the Secretary of Labor which are herein incorporated by reference.

2.  Payrolls and Payroll Records:

a.	Payrolls and basic records relating thereto shall be maintained by the contractor and each subcontractor during the course of the work and preserved for a period of 3 years from the date of completion of the contract for all laborers, mechanics, apprentices, trainees, watchmen, helpers, and guards working at the site of the work.

b.	The payroll records shall contain the name, social security number, and address of each such employee; his or her correct classification; hourly rates of wages paid (including rates of contributions or costs anticipated for bona fide fringe benefits or cash equivalent thereof the types described in Section 1(b)(2)(B) of the Davis Bacon Act); daily and weekly number of hours worked; deductions made; and actual wages paid.  In addition, for Appalachian contracts, the payroll records shall contain a notation indicating whether the employee does, or does not, normally reside in the labor area as defined in Attachment A, paragraph 1.  Whenever the Secretary of Labor, pursuant to Section IV, paragraph 3b, has found that the wages of any laborer or mechanic include the amount of any costs reasonably anticipated in providing benefits under a plan or program described in Section 1(b)(2)(B) of the Davis Bacon Act, the contractor and each subcontractor shall maintain records which show that the commitment to provide such benefits is enforceable, that the plan or program is financially responsible, that the plan or program has been communicated in writing to the laborers or mechanics affected, and show the cost anticipated or the actual cost incurred in providing benefits.  Contractors or subcontractors employing apprentices or trainees under approved programs shall maintain written evidence of the registration of apprentices and trainees, and ratios and wage rates prescribed in the applicable programs.

c. Each contractor and subcontractor shall furnish, each week in which any contract work is performed, to the SHA resident engineer a payroll of wages paid each of its employees (including apprentices, trainees, and helpers, described in Section IV, paragraphs 4 and 5, and watchmen and guards engaged on work during the preceding weekly payroll period).  The payroll submitted shall set out accurately and completely all of the information required to be maintained under paragraph 2b of this Section V.  This information may be submitted in any form desired.  Optional Form WH-347 is available for this purpose and may be purchased from the Superintendent of Documents (Federal stock number 029-005-0014-1), U.S. Government Printing Office, Washington, D.C. 20402.  The prime contractor is responsible for the submission of copies of payrolls by all subcontractors.

d. Each payroll submitted shall be accompanied by a "Statement of Compliance," signed by the contractor or subcontractor or his/her agent who pays or supervises the payment of the persons employed under the contract and shall certify the following:

(1) that the payroll for the payroll period contains the information required to be maintained under paragraph 2b of this Section V and that such information is correct and complete;

(2) that such laborer or mechanic (including each helper, apprentice, and trainee) employed on the contract during the payroll period has been paid the full weekly wages earned, without rebate, either directly or indirectly, and that no deductions have been made either directly or indirectly from the full wages earned, other than permissible deductions as set forth in the Regulations, 29 CFR 3;

(3) that each laborer or mechanic has been paid not less that the applicable wage rate and fringe benefits or cash equivalent for the classification of worked performed, as specified in the applicable wage determination incorporated into the contract.

e.  The weekly submission of a properly executed certification set forth on the reverse side of Optional Form WH-347 shall satisfy the requirement for submission of the "Statement of Compliance" required by paragraph 2d of this Section V.

f.  The falsification of any of the above certifications may subject the contractor to civil or criminal prosecution under 18 U.S.C. 1001 and 31 U.S.C. 231.

g.  The contractor or subcontractor shall make the records required under paragraph 2b of this Section V available for inspection, copying, or transcription by authorized representatives of the SHA, the FHWA, or the DOL, and shall permit such representatives to interview employees during working hours on the job.  If the contractor or subcontractor fails to submit the required records or to make them available, the SHA, the FHWA, the DOL, or all may, after written notice to the contractor, sponsor, applicant, or owner, take such actions as may be necessary to cause the suspension of any further payment, advance, or guarantee of funds.  Furthermore, failure to submit the required records upon request or to make such records available may be grounds for debarment action pursuant to 29 CFR 5.12.

VI.  RECORD OF MATERIALS, SUPPLIES, AND LABOR

1.	On all Federal-aid contracts on the National Highway System, except those which provide solely for the installation of protective devices at railroad grade crossings, those which are constructed on a force account or direct labor basis, highway beautification contracts, and contracts for which the total final construction cost for roadway and bridge is less than $1,000,000 (23 CFR 635) the contractor shall:

a.	Become familiar with the list of specific materials and supplies contained in Form FHWA-47, "Statement of Materials and Labor Used by Contractor of Highway Construction Involving Federal Funds," prior to the commencement of work under this contract.

b.	Maintain a record of the total cost of all materials and supplies purchased for and incorporated in the work, and also of the quantities of those specific materials and supplies listed on Form FHWA-47, and in the units shown on Form FHWA-47.

c.	Furnish, upon the completion of the contract, to the SHA resident engineer on Form FHWA-47 together with the data required in paragraph 1b relative to materials and supplies, a final labor summary of all contract work indicating the total hours worked and the total amount earned.

2.  At the prime contractor's option, either a single report covering all contract work or separate reports for the contractor and for each subcontract shall be submitted.

VII.  SUBLETTING OR ASSIGNING THE CONTRACT

1.  The contractor shall perform with its own organization contract work amounting to not less than 30 percent (or a greater percentage if specified elsewhere in the contract) of the total original contract price, excluding any specialty items designated by the State.  Specialty items may be performed by subcontract and the amount of any such specialty items performed may be deducted from the total original contract price before computing the amount of work required to be performed by the contractor's own organization (23 CFR 635).

a.	"Its own organization" shall be construed to include only workers employed and paid directly by the prime contractor and equipment owned or rented by the prime contractor, with or without operators.  Such term does not include employees or equipment of a subcontractor, assignee, or agent of the prime contractor.

b.	"Specialty Items" shall be construed to be limited to work that requires highly specialized knowledge, abilities, or equipment not ordinarily available in the type of contracting organizations qualified and expected to bid on the contract as a whole and in general are to be limited to minor components of the overall contract.

2.  The contract amount upon which the requirements set forth in paragraph 1 of Section VII is computed includes the cost of material and manufactured products which are to be purchased or produced by the contractor under the contract provisions.

13

3.  The contractor shall furnish (a) a competent superintendent or supervisor who is employed by the firm, has full authority to direct performance of the work in accordance with the contract requirements, and is in charge of all construction operations (regardless of who performs the work) and (b) such other of its own organizational resources (supervision, management, and engineering services) as the SHA contracting officer determines is necessary to assure the performance of the contract.

4.  No portion of the contract shall be sublet, assigned or otherwise disposed of except with the written consent of the SHA contracting officer, or authorized representative, and such consent when given shall not be construed to relieve the contractor of any responsibility for the fulfillment of the contract.  Written consent will be given only after the SHA has assured that each subcontract is evidenced in writing and that it contains all pertinent provisions and requirements of the prime contract.

VIII.  SAFETY: ACCIDENT PREVENTION

1.  In the performance of this contract the contractor shall comply with all applicable Federal, State, and local laws governing safety, health, and sanitation (23 CFR 635).  The contractor shall provide all safeguards, safety devices and protective equipment and take any other needed actions as it determines, or as the SHA contracting officer may determine, to be reasonably necessary to protect the life and health of employees on the job and the safety of the public and to protect property in connection with the performance of the work covered by the contract.

2.  It is a condition of this contract, and shall be made a condition of each subcontract, which the contractor enters into pursuant to this contract, that the contractor and any subcontractor shall not permit any employee, in performance of the contract, to work in surroundings or under conditions which are unsanitary, hazardous or dangerous to his/her health or safety, as determined under construction safety and health standards

2.	Instructions for Certification - Lower Tier Covered

Transactions:

(Applicable to all subcontracts, purchase orders and other lower tier transactions of $25,000 or more - 49 CFR 29)

a.  By signing and submitting this proposal, the prospective lower tier is providing the certification set out below.

b.  The certification in this clause is a material representation of fact upon which reliance was placed when this transaction was entered into.  If it is later determined that the prospective lower tier participant knowingly rendered an erroneous certification, in addition to other remedies available to the Federal Government, the department, or agency with which this transaction originated may pursue available remedies, including suspension and/or debarment.

c.  The prospective lower tier participant shall provide immediate written notice to the person to which this proposal is submitted if at any time the prospective lower tier participant learns that its certification was erroneous by reason of changed circumstances.

d.  The terms "covered transaction," "debarred," "suspended," "ineligible," "primary covered transaction," "participant," "person," "principal," "proposal," and "voluntarily excluded," as used in this clause, have the meanings set out in the Definitions and Coverage sections of rules implementing Executive Order 12549.  You may contact the person to which this proposal is submitted for assistance in obtaining a copy of those regulations.

e.  The prospective lower tier participant agrees by submitting this proposal that, should the proposed covered transaction be entered into, it shall not knowingly enter into any lower tier covered transaction with a person who is debarred, suspended, declared ineligible, or voluntarily excluded from participation in this covered transaction, unless authorized by the department or agency with which this transaction originated.

f.  The prospective lower tier participant further agrees by submitting this proposal that it will include this clause titled "Certification Regarding Debarment, Suspension, Ineligibility and Voluntary Exclusion-Lower Tier Covered Transaction," without modification, in all lower tier covered transactions and in all solicitations for lower tier covered transactions.

g.  A participant in a covered transaction may rely upon a certification of a prospective participant in a lower tier covered transaction that is not debarred, suspended, ineligible, or voluntarily excluded from the covered transaction, unless it knows that the certification is erroneous.  A participant may decide the method and frequency by which it determines the eligibility of its principals.  Each participant may, but is not required to, check the Non-procurement List.

h.  Nothing contained in the foregoing shall be construed to require establishment of a system of records in order to render in good faith the certification required by this clause.  The knowledge and information of participant is not required to exceed that which is normally possessed by a prudent person in the ordinary course of business dealings.

i.  Except for transactions authorized under paragraph e of these instructions, if a participant in a covered transaction knowingly enters into a lower tier covered transaction with a person who is suspended, debarred, ineligible, or voluntarily excluded from participation in this transaction, in addition to other remedies available to the Federal Government, the department or agency with which this transaction originated may pursue available remedies, including suspension and/or debarment.

* * * * *

Certification Regarding Debarment, Suspension, Ineligibility and Voluntary Exclusion--Lower Tier Covered Transactions:

1.  The prospective lower tier participant certifies, by submission of this proposal, that neither it nor its principals is presently debarred, suspended, proposed for debarment, declared ineligible, or voluntarily excluded from participation in this transaction by any Federal department or agency.

2.  Where the prospective lower tier participant is unable to certify to any of the statements in this certification, such prospective participant shall attach an explanation to this proposal.

* * * * *

XII.  CERTIFICATION REGARDING USE OF CONTRACT

FUNDS FOR LOBBYING

(Applicable to all Federal-aid construction contracts and to all related subcontracts which exceed $100,000 - 49 CFR 20)

1.  The prospective participant certifies, by signing and submitting this bid or proposal, to the best of his or her knowledge and belief, that:

a.  No Federal appropriated funds have been paid or will be paid, by or on behalf of the undersigned, to any person for influencing or attempting to influence an officer or employee of any Federal agency, a Member of Congress, an officer or employee of Congress, or an employee of a Member of Congress in connection with the awarding of any Federal contract, the making of any Federal grant, the making of any Federal loan, the entering into of any cooperative agreement, and the extension, continuation, renewal, amendment, or modification of any Federal contract, grant, loan, or cooperative agreement.

b.  If any funds other than Federal appropriated funds have been paid or will be paid to any person for influencing or attempting to influence an officer or employee of any Federal agency, a Member of Congress, an officer or employee of Congress, or an employee of a Member of Congress in connection with this Federal contract, grant, loan, or cooperative agreement, the undersigned shall complete and submit Standard Form-LLL, "Disclosure Form to Report Lobbying," in accordance with its instructions.

2.  This certification is a material representation of fact upon which reliance was placed when this transaction was made or entered into.  Submission of this certification is a prerequisite for making or entering into this transaction imposed by 31 U.S.C. 1352.  Any person who fails to file the required certification shall be subject to a civil penalty of not less than $10,000 and not more than $100,000 for each such failure.

3.  The prospective participant also agrees by submitting his or her bid or proposal that he or she shall require that the language of this certification be included in all lower tier subcontracts, which exceed $100,000 and that all such recipients shall certify and disclose accordingly.

14

AFFIRMATIVE ACTION REQUIREMENTS

UTILIZATION OF DISADVANTAGED BUSINESS ENTERPRISES

FOR FEDERAL-AID CONTRACTS

CONTRACT GOALS

FOR THE PURPOSE OF THIS CONTRACT, A GOAL OF 5% PERCENT HAS BEEN ESTABLISHED FOR SOCIALLY AND ECONOMICALLY DISADVANTAGED BUSINESSES THAT ARE OWNED AND CONTROLLED BY – THOSE INDIVIDUALS WHO ARE BLACK AMERICANS, HISPANIC AMERICANS, ASIAN-PACIFIC AMERICANS, SUBCONTINENT ASIAN AMERICANS, NATIVE AMERICANS, OR WOMEN PURSUANT TO THE MARYLAND DEPARTMENT OF TRANSPORTATION (MDOT) MINORITY BUSINESS ENTERPRISE PROGRAM:

It is the policy of the Maryland Department of Transportation that disadvantaged business enterprises as defined in 49 CFR Part 26 and the Safe, Accountable, Flexible, Efficient Transportation Equity Act: A Legacy for Users (SAFETEA-LU) shall have an equal opportunity to participate in the performance of the contracts financed in whole or in part with Federal funds under these agreements.  Consequently, the disadvantaged business enterprise requirements of 49 CFR Part 26 and SAFETEA-LU apply to this agreement.

The bidder agrees to ensure that disadvantaged business enterprises as defined in 49 CFR Part 26 and SAFETEA-LU have an equal opportunity to participate in the performance of contracts and subcontracts financed in whole or in part with Federal funds provided under this agreement.  In this regard, all bidders shall take all necessary and reasonable steps in accordance with 49 CFR Part 26 and SAFETEA-LU to ensure that disadvantaged business enterprises have an equal opportunity to compete for and perform on Federally funded contracts.  The Maryland Department of Transportation and their bidders shall not discriminate on the basis of race, color, national origin, or sex in the award and performance of this Contract.

- GENERAL

For the purpose of these requirements, the following terms as defined below shall apply:

Administration Representative – A DBE/MBE Officer or employee of an Administration who enforces the laws and regulations pertaining to disadvantaged and minority business enterprise and contract compliance.

Affirmative Actions – Specific steps taken to eliminate discrimination and its effects, to ensure nondiscriminatory results and practices in the future, and to involve disadvantaged and minority business enterprises fully in contracts and programs.

Business Enterprises – Any legal entity which is organized in any form other than as a joint venture (e.g., sole proprietorship, partnership, corporation, etc.) to engage in lawful commercial transactions.

15

Certified Business – A business which by order of the Chair/MBE Advisory Council or his/hers designee, has been certified as a bona fide DBE/MBE.  MDOT certification does not equate to a pre-qualification status.

DBE – Disadvantaged Business Enterprise – Reference 49 CFR, Part 26, Subpart A) a small business concern: (1) which is at least 51 percent owned by one or more socially and economically disadvantaged individuals.  Where stock ownership is involved, the disadvantaged owner(s) shall own at least 51 percent of each class of voting stock and at least 51 percent of the aggregate of all classes of stock that have been issued (also applies to publicly owned businesses); and (2) whose management and daily business operations are controlled by one or more of the socially and economically disadvantaged individuals who have ownership.  In this specification the terms MBE and DBE have the same meaning.

DBE/MBE Directory – A compilation of businesses certified by MDOT as disadvantaged, minority, or socially and economically disadvantaged businesses.  The directory will be published annually with quarterly supplements.  It will also be provided in automated format and on the Internet to be updated as changes are made.

DBE/MBE Participation Packet – The documents submitted by the bidder or proposer pursuant to the appropriate special bid provisions.  The DBE/MBE Participation Packet consists of the Certified DBE Utilization and Fair Solicitation Affidavit and the DBE Participation Schedule, both of which must be submitted with your bid or initial price proposal.  The DBE Participation Packet also includes the following documents, which shall be submitted after bids or proposals are opened: Outreach Efforts Compliance Statement (MDOT-OP-018-2), DBE Subcontractor Project Participation Affidavit (MDOT-OP-019-2), MDOT Joint Venture Disclosure Affidavit (D-EEO-006), and Minority Contractor Unavailability Certificate (OOC46).

DBE/MBE Program – A program developed by MDOT to implement the requirements of Title 14, Subtitle 3 of the State Finance and Procurement Article, Annotated Code of Maryland; Title 10, Subtitle 3 of the State Finance and Procurement Article of the Annotated Code of Maryland for Leases of State-Owned Property; and 49 CFR, Part 26, Subparts A and C for all Federal Department of Transportation Financial Assistance Programs.

Director, Office of Equal Opportunity – The individual designated for the Administration’s overall MBE compliance.

Joint Venture – An association of a DBE/MBE firm and one or more other firms to carry out a single, for-profit business enterprise, for which the parties combine their property, capital, efforts, skills, and knowledge, and in which the DBE/MBE is responsible for a distinct, clearly defined portion of the work of the contract and whose share in the capital contribution, control, management, risks, and profits of the joint venture are commensurate with its ownership interest.

Small Business Administration (SBA) 8(a) Certification – The SBA 8(a) Certification Program is a Federal Program which establishes firms as disadvantaged and eligible for participation in the Federal SBA Program.

Socially and Economically Disadvantaged Individual Pursuant to 49 CFR, Part 26 – Those individuals who are citizens of the United States (or lawfully admitted permanent residents).  For convenience, these individuals and groups are referred to as “minorities” in this document and who are:

16

1.	Found by the MDOT to be socially and economically disadvantaged on a case-by-case basis;

2.	Any individual in the following groups, members of which are refutably presumed to be socially and economically disadvantaged.

a.	“Black Americans,” which includes persons having origins in any of the Black racial groups of Africa;

b.	“Hispanic Americans,” which includes persons of Mexican, Puerto Rican, Cuban, Dominican, Central or South American, or other Spanish or Portuguese culture or origin, regardless of race;

c.	“Native Americans,” which includes persons who are American Indian, Eskimos, Aleuts, or Native Hawaiians;

d.	“Asian-Pacific Americans,” which included persons whose origins are from Japan, China, Taiwan, Korea, Burma (Myanmar), Vietnam, Laos, Cambodia (Kampuchea), Thailand, Malaysia, Indonesia, the Philippines, Brunei, Samoa, Guam, the U.S. Trust Territories of the Pacific Islands (Republic of Palau), the Commonwealth of the Northern Marianas Islands, Macao, Fiji, Tonga, Kirbati, Juvalu, Nauru, Federated States of Micronesia, or Hong Kong;

e.	“Subcontinent Asian American,” which includes persons whose origins are from India, Pakistan, Bangladesh, Bhutan, the Maldives Islands, Nepal or Sri Lanka;

f.	Women;

3.	Only those persons whose personal net worth does not exceed $750,000 may be found to be economically disadvantaged.

- DBE/MBE and Good Faith Effort Requirements

1.	This contract includes a DBE participation goal for subcontracting and/or procurement of materials and/or services.  Bidders (or offerors) must make a good faith effort to meet the DBE participation goal before bids or proposals are due, including outreach efforts.  A bid or initial proposal must include both a completed and executed Certified DBE Utilization and Fair Solicitation Affidavit and DBE Participation Schedule.  The failure of a bidder to complete and submit these documents shall result in a determination that the bid is not responsive.  The failure of an offeror to complete and submit these documents shall result in a determination that the proposal is not susceptible of being selected for award.

2.	In making a good faith effort to achieve the DBE goal, prior to completing the Certified DBE Utilization and Fair Solicitation Affidavit and the DBE Participation Schedule and prior to submitting a bid or initial proposal the bidders (or offerors) including those bidders or offerors that are certified DBEs must:

a.	Identify specific work categories within the scope of the procurement appropriate for subcontracting and/or procurement of materials and/or services;

b.	Solicit DBEs in writing at least 10 days before bids or initial proposals are due, describing the identified work categories and providing instructions on how to bid on the subcontracts and/or procurement of materials and/or services;

17

c.	Attempt to make personal contact with the DBEs solicited and to document these attempts;

d.	Assist DBEs to fulfill, or to seek waiver of, bonding requirements; and

e.	Attend prebid or other meetings the procurement agency schedules to publicize contracting opportunities to DBEs.

3.	All firms bidding on a Federal-Aid Contract shall submit the name and address of all subcontractors, service providers and suppliers that submitted quotes on the Contract.  All subcontractors, service providers and suppliers shall complete and submit the form entitled Contractor Information, to the Administration.

4.	The bidder shall seek commitments from disadvantaged business enterprises by subcontracting and/or procurement of materials and/or services, the combined value of which equals or exceeds the appropriate percent (goal) of the total value of the prime contract.  A bidder may count toward its DBE goals expenditures for materials and supplies obtained from DBE regular dealers and/or manufacturers provided that the DBEs assume the actual and contractual responsibility for the provision of the materials and supplies.  The bidder may count its entire expenditure to a DBE manufacturer (i.e., a supplier that produces goods from raw materials or substantially alters them before resale).  The bidder may count sixty (60) percent of its expenditures to a DBE regular dealer that is not a manufacturer, provided that the DBE supplier performs a commercially useful function in the supply process.  The apparent low bidder shall submit to the Administration, within ten (10) business days after notification that it is the apparent low bidder, an acceptable Affirmative Action Plan for the utilization of Disadvantaged Business Enterprises in this Contract.  The Contract will not be awarded without the Bidder’s AAP being approved by the Administration.

The Affirmative Action Plan shall include as a minimum:

  - The name of an employee designated as the bidder’s liaison officer for minority affairs.

  - A complete DBE Subcontractor Project Participation Affidavit (MDOT-OP-019-2), using contractors whose names appear in the DBE/MBE directory or who are otherwise certified by MDOT as being a disadvantaged business enterprise.  Except as permitted by law and approved by the Administration, this affidavit shall include all DBE firms identified on the DBE Participation Schedule with a percentage of participation that meets or exceeds the percentage of participation indicated in the bid or initial proposal.

  - A completed Outreach Efforts Compliance Statement (MDOT-OP 018-2).

5.	When a bidder intends to attain the appropriate goal for disadvantaged business enterprise participation by use of a joint venture, the bidder shall submit a Joint Venture Disclosure Affidavit (Form D-EEO-006-A) showing the extent of disadvantaged business participation.  If a bidder intends to use a joint venture as a subcontractor to meet its goal, the affidavit shall be submitted through the bidder by the proposed subcontractor and be signed by all parties.  A DBE, even in a joint venture arrangement shall be certified as a DBE by MDOT prior to being included in the Affirmative Action Plan.

6.	Where the proposed DBE participation does not meet the DBE contract goals, sufficient evidence to demonstrate that the bidder has taken all necessary and reasonable steps to make a good faith effort to meet these goals shall be required.

18

7.	Determination of Bid Responsiveness for Federal-Aid Contracts

If the bidder is unable to secure from DBEs by subcontracting and/or by procurement of materials and/or services, commitments which at least equal the appropriate percent (goal) of the values of the prime Contract at the time of bid, he shall request, in writing, a waiver of the unmet portion of the goal.  This request must be initiated by checking the appropriate box on the Certified DBE Utilization and Fair Solicitation Affidavit submitted with the bid or initial proposal.

The waiver may be granted by the Administrator.  To obtain approval of a waiver, the bidder shall submit the following information:

a.	   A detailed statement of efforts made prior to bid to contact and negotiate with DBEs including:  (i) the dates, names, addresses, and telephone numbers of DBEs who were contacted; (ii) a description of the information provided to DBEs requesting the plans, specifications, and anticipated time schedule for portions of the work to be performed and (iii) a detailed statement of the reasons why additional prospective agreements with DBEs were not reached;

b. 	A detailed statement of the efforts made to select portions of the work proposed to be performed by DBEs in order to increase the likelihood of achieving the stated goals;

c.	   For each DBE that the Contractor considers not qualified, but from which a bid has been received, a detailed statement of the reasons for the bidder’s conclusion; and

d.	   For each DBE contacted but unavailable, (i) a Minority Contractor Unavailability Certificate (Form OOC46) signed by the disadvantaged business enterprise, or (ii) a statement from the bidder shall be submitted that states that the DBE refused to sign the Certificate.

8.		Guidance concerning good faith efforts.  The following is a list of the types of actions and factors that will be used to determine the bidder's or offeror’s good faith efforts to obtain DBE participation. It is not intended to be a mandatory checklist, nor is it intended to be exclusive or exhaustive. Other factors or types of efforts may be relevant in appropriate cases.

(1)	Soliciting through all reasonable and available means (e.g. attendance at pre-bid meetings, advertising and/or written notices) the interest of certified DBEs who have the capability to perform the work of the contract. The bidder must solicit this interest within sufficient time to allow the DBEs to respond to the solicitation. The bidder must determine with certainty if the DBEs are interested by taking appropriate steps to follow up initial solicitations.

(2)	Selecting portions of the work to be performed by DBEs in order to increase the likelihood that the DBE goals will be achieved. This includes, where appropriate, breaking out contract work items into economically feasible units to facilitate DBE participation, even when the bidder or offeror might otherwise prefer to perform these work items with its own forces.

(3)	Providing interested DBEs with adequate information about the plans, specifications, and requirements of the contract in a timely manner to assist them in responding to a solicitation.

19

(4)	(a) Negotiating in good faith with interested DBEs. It is the bidder's or offeror’s responsibility to make a portion of the work available to DBE subcontractors and suppliers and to select those portions of the work or material needs consistent with the available DBE subcontractors and suppliers, so as to facilitate DBE participation.

(b) A bidder using good business judgment would consider a number of factors in negotiating with subcontractors, including DBE subcontractors, and would take a firm's price and capabilities as well as contract goals into consideration. However, the fact that there may be some additional costs involved in finding and using DBEs is not in itself sufficient reason for a bidder's failure to meet the contract DBE goal, as long as such costs are reasonable. Also, the ability or desire of a prime contractor to perform the work of a contract with its own organization does not relieve the bidder of the responsibility to make good faith efforts. Bidders and offerors are not, however, required to accept higher quotes from DBEs if the price difference is excessive or unreasonable.

(5)	Not rejecting DBEs as being unqualified without sound reasons based on a thorough investigation of their capabilities. The contractor's standing within its industry, membership in specific groups, organizations, or associations and political or social affiliations (for example union vs. non-union employee status) are not legitimate causes for the rejection or non-solicitation of bids in the contractor's efforts to meet the project goal.

(6)	Making efforts to assist interested DBEs in obtaining bonding, lines of credit, or insurance as required by the recipient or contractor.

(7)	Making efforts to assist interested DBEs in obtaining necessary equipment, supplies, materials, or related assistance or services.

(8)	Effectively using the services of available minority/women community organizations; minority/women contractors' groups; local, state, and Federal minority/women business assistance offices; and other organizations as allowed on a case-by-case basis to provide assistance in the recruitment and placement of DBEs.

20

(9)	 In determining whether a bidder or offeror has made good faith efforts, you may take into account the performance of other bidders or offerors in meeting the contract goal. For example, when the apparent successful bidder or offeror fails to meet the contract goal, but others meet it, the Administration may reasonably raise the question of whether, with additional reasonable efforts, the apparent successful bidder or offeror could have met the goal. If the apparent successful bidder or offeror fails to meet the goal, but meets or exceeds the average DBE participation obtained by other bidders or offerors, the Administration may view this, in conjunction with other factors, as evidence of the apparent successful bidder or offeror having made good faith efforts.

9.	Bidder Use of DBE Special Services

The bidder shall consider, whenever possible, utilizing the services of minority-owned banks.  Most minority banks are full-service corporations that can provide an array of financial services such as Treasury and Tax Loan fund accounts, time and demand deposit accounts, payroll services, and if needed, organization investment counseling.

10.	Bidder Records

The bidder shall maintain records showing actions which have been taken to comply with procedures set forth herein.

11.	Bidder Cooperation

The bidder shall cooperate with the Administration Representative in any reviews of the Contractor’s procedures and practices with respect to DBEs which the Administration Representative may from time to time conduct.

12.	Bidder DBE Modifications

During the life of the Contract, all plans to modify the approved DBE participation program will require the approval of the Administrator or his authorized representative.  This shall include any changes to the items of work to be sublet or materials and services to be obtained which differ for those in the original DBE participation program.  Any such request for revisions shall be directed to the appropriate District Engineer for their disposition.

C.	RECORDS AND REPORTS

1.	The Contractor shall keep such records as are necessary to determine compliance with its Minority Business Enterprise utilization obligations.  The records kept by the Contractor shall be designed to indicate:

a.	The name of disadvantaged and non-disadvantaged subcontractors and suppliers, the type of work materials or services being performed on or incorporated in this project, and the monetary value of such work materials or services.

- Documentation of all correspondence, contacts, telephone calls, etc., to obtain the services of disadvantaged business enterprises on this project.

21

c.	The progress and efforts made in seeking out disadvantaged contractor organizations and individual disadvantaged contractors for work on this project.

2.	Information required to be submitted for Federally Assisted contracts in accordance with 49 CFR Part 26:

a.	All bidders (not only the apparent successful bidder) shall provide the following information:

- The age of the bidding firm; and

(2)	The annual gross receipts of the bidding firm.

b.	All bidders (not only the apparent successful bidder) shall provide the following information for each firm quoting or considered as subcontractors:

(1)	The name of firm; and

(2)	The address of firm.

c.	The Administration will contact each of the firms quoting or considered as subcontractors to obtain:

(1)	The age of the firm; and

(2)	The annual gross receipts of the firm

If this information already has been gathered by the Administration on a firm and it is current, it will not be requested.

3.	The Contractor shall submit reports on a quarterly basis of those contracts and other business transactions executed with disadvantaged business enterprises with respect to the records referred to in Subparagraph 1.a above, in such form, manner, and content as prescribed by the Administration.  The quarterly reports shall be due on the 15th calendar day of January, April, July, and October.  If the Contractor cannot submit their report on time, they shall notify the Administration’s Representative and request additional time to submit the report.  Failure of the Contractor to report in a timely manner may result in a finding of noncompliance.  Additional reports may be required by the Administration upon written request.

4.	To ensure compliance with the certified DBE contract participation goals, the Contractor shall:

a.	Submit monthly, a report listing unpaid invoices, over 30 days, from all certified DBE subcontractors and the reason payment has not been made;

b.	Include in its agreement with certified DBE subcontractors a requirement that the DBE subcontractors are to submit monthly to the Administration, a report identifying the prime Contractor and listing the following:

      - Payment received from the Contractor in the preceding 30 days; and

      - Invoices for which the subcontractor has not been paid.

22

5.	All such records shall be retained for a period of three years following acceptance of final payment and shall be available for inspection by the U.S. Department of Transportation, the Maryland Department of Transportation, and the Administration.

D.	ADMINISTRATIVE PROCEDURES FOR ENFORCEMENT

1.	Whenever the Administration believes the prime Contractor or any subcontractor may not be operating in compliance with the terms of these provisions, the Administration Representative will conduct an investigation.  If the Administration Representative finds the prime Contractor or any subcontractor not in compliance with these provisions, he will make a report of non-compliance and notify such Contractor in writing of the steps that will, in the judgment of the Administration, bring the Contractor into compliance.  If the Contractor fails or refuses to comply fully with such steps, the Administration Representative will make a final report of noncompliance to the Administrator, who may direct the imposition of one or more of the sanctions listed below:

a.	Suspension of work on a project, pending correction;

b.	Withholding payment or a percentage thereof, pending correction;

c.	Referral of DBE/MBE to MDOT Office of MBE for review for decertification or minority business fraud investigation;

d.	Referral to MDOT Office of MBE for review/referral to the Attorney General’s Office for review for initiation of debarment;

e.	Referral to the Attorney General’s Office for review for debarment or for criminal prosecution through the MDOT Office of General Counsel; or

f.	Any other action as appropriate.

The Administrator will determine which sanction(s) should be imposed in order to promote the purpose of the MDOT DBE/MBE Program.

2.	If the documents used to determine the status of a DBE contain false, misleading, or misrepresenting information, the matter may be referred to the MDOT Office of MBE for appropriate action.

3.	Loss of DBE Certification

a.	When a prime Contractor has made a commitment to use a DBE who has lost its certification but the subcontract has not been executed prior to the notice of loss of certification, the prime Contractor is required to obtain an eligible, certified DBE for the contract or demonstrate to MDOT that it has made a good faith effort to do so.

b.	When a prime Contractor has executed a contract with a DBE subcontractor before the notice of loss of certification, the prime Contractor may continue to use the firm on the contract and may continue to receive credit towards its DBE goal, i.e., contract goal, for the work of that subcontractor.

23

c.	The work carried out by a DBE Prime Contractor would be counted by MDOT up to the loss of certification.  The work performed after the loss of certification would not be considered DBE participation.

d.	When a DBE subcontractor has lost its certification, MDOT may not continue to count the DBE participation which takes place after the loss of certification as DBE work when counting participation towards the overall goal of the modal administration or the Department.

- If a DBEs loss of certification is the result of exceeding the size standards while performing on a contract, the DBE participation may be counted for both the contract goal and the overall goal.

E.	SUBCONTRACTING.

Subcontracting by the Prime Contractor.  Form B Request for Approval of Subcontractor shall be used by the Prime Contractor to request approval of a Subcontractor and also to ensure that a formal Subcontract has been or will be written and kept on file by the Prime Contractor.  Completion and submittal of the form by the Prime Contractor acknowledges that the Administration’s Contracting Officer may require the submission of the written Subcontract for review by the Administration and/or FHWA.

Lower Tier Subcontracting by an Approved Subcontractor.  Form B Subcontractor’s Request for Approval of Lower Tier Subcontractor shall be used by an Approved Subcontractor to request approval of a Lower Tier Subcontractor and also to ensure that a formal Subcontract has been or will be written and kept on file by the Subcontractor.  Completion and submittal of the form by the Subcontractor acknowledges that the Administration’s Contracting Officer may require the submission of the written Subcontract for review by the Administration and/or FHWA.

Form Acquisitions.  Maryland State Highway Administration Form B may be acquired through the Administration’s Contracts Award Team or District Office.  All questions should be directed to the Office of Construction, Contracts Award Team.

It is the Administration’s intention to randomly select during each calendar quarter a representative sample of written Subcontracts for review.  This review will be conducted by the Office of Construction's Contracts Award Team.

24

MBE/DBE COMPLIANCE FIELD MEETING

A MBE/DBE compliance Field Meeting will be conducted to review the responsibilities of the Administration and the Contractor’s personnel relative to MBE/DBE Compliance and documentation.  The meeting will be held within two weeks after starting work on the project.

The Construction Project Engineer, who will notify the following of the date, time and location, will arrange the meeting.  At least one week advanced notice will be required.

(a)	Administrative Representatives.

(1)	Director, Office of Equal Opportunity or Designee

(2)	District Equal Opportunity Officer

(3)	Regional Constructional Engineer

(4)	Construction Project Engineer

(5)	Construction Inspection Division Inspector

(b)	Contract Representatives.

(1)	Superintendent - Prime Contractor

(2)	Equal Opportunity Officer - Prime Contractor

(3)	Owner/Superintendent/Foreman MBE/ DBE - Subcontractor

The Construction Project Engineer and Equal Opportunity Representative will jointly conduct the meeting.  The Contractor shall notify the appropriate subcontractors and ensure their attendance.

25

TRAFFIC CONTROL PLAN CERTIFICATION

PRIOR TO THE COMMENCEMENT OF WORK ON THIS PROJECT, THE SUCCESSFUL BIDDER WILL BE REQUIRED TO COMPLETE A TRAFFIC CONTROL PLAN CERTIFICATION, CONTAINING THE INFORMATION SHOWN BELOW.  THE CERTIFICATION FORM WILL BE PROVIDED TO THE SUCCESSFUL BIDDER UPON AWARD OF THE CONTRACT.

The Administration's Traffic Control Plan (TCP) has been reviewed and the following course of action shall be followed:

Option 1

The TCP is accepted and shall be used on this project.

Option 2

The TCP is accepted; however, revisions and/or additions shall be submitted for approval in conformance with the Administration's Specifications 104.01.

Option 3

The TCP is not accepted and revision shall be submitted for approval in accordance with the Administration's Specifications 104.01.

It is understood that the effective implementation of the approved TCP is the responsibility of the Contractor.  Minor modifications may be made by the Traffic Manager if field conditions warrant and prior concurrence is obtained from the Engineer.  Significant changes to the TCP will be submitted to the Engineer in writing, for approval, in conformance with the Administration's Specifications 104.01.

(DATE)	(SIGNATURE)

(PRINT SIGNATURE)

(TITLE)

26

# PREVAILING WAGE

INSTRUCTIONS FOR THE CONTRACTOR

PAYROLLS.

Non-Federally Funded Contracts.  For Non-Federally funded projects, which include prevailing wage rates, the prime Contractor and each subcontractor shall submit two copies of their payroll records on a weekly basis.  Two copies shall be submitted to Phyllis Grover, Project Manager,  City of Aberdeen, 60 North Parke Street, Aberdeen, MD 21001, where they will be available for inspection during business hours.  All wages shall be paid in conformance with the State Finance and Procurement Article, Section 17-201-17-226 of the Annotated Code of Maryland and the Fair Labor Standards Amendments of 1974 (P.L. 93259).  If the award amount of a Non-Federally funded job is less than $500,000, the project will be exempt from prevailing wage requirements.

A review has been made of the wage conditions in the locality and, based on the information available, the wage rates and fringe payments listed are determined by the Commissioner of the Department of Labor and Industry to be prevailing for the Contract for the described classes of labor in conformance with the law.  It shall be the responsibility of the Contractor to fully comply with the law and to contact the Office of the Commissioner of Labor and Industry for interpretation of the provisions of the law.

Federally Funded Contracts.  For Federally funded projects, the prime Contractor and each subcontractor shall submit one copy of the certified payroll to the Project Manager.

General Requirements for Federally and Non-Federally Funded Contracts.  All payrolls are subject to the following requirements:

(a)	All payrolls shall be numbered, beginning at No. 1, and consecutively numbered through the end of the Contract.

(b)	Contract and FAP numbers shall be shown on all payrolls (as applicable).

(c)	All payrolls shall include the employees’ full name, classification, social security number, and home address.

(d)	All payrolls shall show the employee’s basic hourly wage rate, overtime rate (if applicable), and the number of hours worked (tabulated both daily and weekly).

(e)	When fringe benefits are required, indicate separately the amount of employer contributions to fringe benefit funds and/or programs.  The fringe benefits shall be individually identified, but may be tabulated on a separate sheet.  When required fringe benefits are paid in cash, add the required fringe benefit amount to the basic hourly rate to obtain the total prevailing wage rate for the employee.

(f)	The employee’s net pay and the itemized deductions shall be included in all payrolls.

27

(g)	A Contractor may make deductions that are required by law or required by a collective bargaining agreement (between the Contractor and a bona fide labor organization). Deductions are also permitted if they are identified in a written agreement between the employee and employer that was made at the beginning of employment, provided that the Contractor presents the agreement to the Administration before the employee begins working on the Contract.  Each payroll shall also include the U.S. Department of Labor and Hour Public Contracts Division Statement of Compliance Form WH-347 (or its equivalent), signed by an appropriate official of the Contractor/subcontractor.  The Contractor’s name, address, and telephone number shall also to be shown.

(h)	On Non-Federally funded projects, all apprentices shall be registered with the Maryland Apprenticeship and Training Council.

(i)	Contractors employing a classification of worker for which a wage rate was not included on the original wage decision, shall submit to the Project Manager, a request for an additional classification and rate prior to the employee’s employment at the project.

(j)	Payrolls for Non-Federally Funded projects shall be submitted weekly after the end of each payroll period.

(k)	Payrolls for Federally Funded projects shall be submitted weekly after the end of each payroll period.

OVERTIME.

Non-Federally Funded Contracts.  Overtime rates shall be paid by the prime Contractors and subcontractors under their Contracts and agreements with their employees, which in no event shall be less than time and a half the prevailing hourly rate of wages for all hours worked in excess of ten hours in any one calendar day or forty hours in any one calendar week and work performed on Sundays and legal holidays.  Fringe benefits shall be paid for all hours worked, including the overtime hours.  However, the fringe benefit amounts may be excluded from the half time premium due as overtime compensation.

Federally Funded Contracts.  Overtime rates shall be paid as specified in Form FHWA 1273. Fringe benefits shall be paid for all hours worked, including the overtime hours.  However, the fringe benefit amounts may be excluded from the half time premium due as overtime compensation.

PENALTIES.

Non-Federally Funded Contracts.  When the Contractor is delinquent in submitting payroll records, processing of partial payment estimates will be held in abeyance, pending receipt of the records.  The Contractor shall be liable to the Administration for liquidated damages in the amount of $10.00 for each calendar day the records are late.

The Contractor shall be liable to the Administration for liquidated damages in the amount of $20.00 for each day that an employee is paid less than the prevailing wage.

Federally Funded Contracts.  When the Contractor is delinquent in submitting payroll records, processing of partial payment estimates will be held in abeyance pending receipt of the records.

28

NOTICE OF ACTIONS REQUIRED FOR AFFIRMATIVE ACTION TO

ENSURE EQUAL EMPLOYMENT OPPORTUNITY (EXECUTIVE ORDER 11246)

1.	The Offeror's or Bidders attention is called to the "Equal Opportunity Clause" and the "Standard Federal Equal Employment Specifications" set forth herein.

2.	The goals and timetables for minority and female participation, expressed in percentage terms for the Contractor's aggregate work force in each trade on all construction work in the covered area, are as noted in Appendix A and B:

These goals are applicable to all the Contractors' construction work (whether or not it is Federal or federally assisted) performed in the covered area.  If the contractor performs construction work in a geographical area located outside of the covered area, it shall apply the goals established for such geographical area where the work is actually performed.  With regard to this second area, the contractor also is subject to the goals for both its federally involved and nonfederally involved construction.

The contractor's compliance with the Executive Order and the regulations in 41 CFR Part 60-4 shall be based on its implementation of the Equal Opportunity Clause, specific affirmative action obligations required by the specifications set forth in 41 CFR 60-4.3(a), and its efforts to meet the goals.  The hours of minority and female employment and training must be substantially uniform throughout the length of contract, and in each trade, and the contractor shall make a good faith effort to employ minorities and women evenly on each of its projects.  The transfer of minority or female employees or trainees from Contractor to Contractor or from project to project for the sole purpose of meeting the contractor's goals shall be a violation of the contract, the Executive Order and the regulations in 41 CFR Part 60-4.  Compliance with the goals will be measured against the total work hours performed.

3.	The Contractor shall provide written notification to the Director of the Office of Federal Contract Compliance Programs within 10 working days of award of any construction subcontract in excess of $10,000 at any tier for construction work under the contract resulting from this notification.  The notification shall list the name, address, and telephone number of the subcontractor; employer identification number of the subcontractor; estimated dollar amount of the subcontract; estimated starting and completion dates of the subcontract; and the geographical area in which the subcontract is to be performed.

4.	As used in this Notice, and in the contract resulting from this solicitation, the "covered area" is noted on appendix B.

STANDARD FEDERAL EQUAL EMPLOYMENT OPPORTUNITY CONSTRUCTION

CONTRACT SPECIFICATIONS (Executive Order 11246)

1.	As used in these specifications:

a.	"Covered area" means the geographical area described in the solicitation from which this contract resulted;

b.	"Director" means Director, Office of Federal Contract Compliance Programs, United States Department of Labor, or any person to whom the Director delegates authority;

33

c.     “Employer identification number” means the Federal Social Security number used on the

Employer's Quarterly Federal Tax Return, U.S. Treasury Department Form 941.

d.	“Minority” includes:

(i)	Black (all persons having origins in any of the Black African racial groups not of Hispanic origin);

(ii)	Hispanic (all persons of Mexican, Puerto Rican, Cuban, Central or South American or other Spanish culture or origin regardless of race);

(iii)	Asian and Pacific Islander (all persons having origins in any of the original people of the Far East, Southeast Asia, the Indian Subcontinent, or the Pacific Islands); and,

(iv)	American Indians or Alaskan Native (all persons having origins in any of the original peoples of North America and maintaining identifiable tribal affiliations through membership and participation or community identification).

2.	Whenever the Contractor, or any Subcontractor at any tier, subcontracts a portion of the work involving any construction trade, it shall physically include in each subcontract in excess of $10,000 the provisions of these  specifications  and the Notice which contains the applicable goals for minority and female participation and which is set forth in the solicitations from which this contract resulted.

3.	If the Contractor is participating (pursuant to 41 CFR 60-4.5) in a Hometown Plan approved by the U.S. Department of Labor in the covered area either individually or through an association, its affirmative action obligations on all work in the Plan area (including goals and timetables) shall be in accordance with that Plan for those trades which have unions participating in the Plan.  Contractors must be able to demonstrate their participation in and compliance with the provisions of any such Hometown Plan.  Each Contractor or Subcontractor participating in an approved Plan is individually required to comply with its obligations under the EEO clause, and to make a good faith effort to achieve each goal under the Plan in each trade in which it has employees.  The overall good faith performance by other Contractors or Subcontractors toward a goal in an approved Plan does not excuse any covered Contractor's or Subcontractor's failure to take good faith efforts to achieve the Plan goals and timetables.

4.	The Contractor shall implement the specific affirmative action standards provided in paragraphs 7.a through 7.p of these specifications.  The goals set forth in the solicitation from which this contract resulted are expressed as percentages of the total hours of employment and training of minority and female utilization the Contractor should reasonably be able to achieve in each construction trade in which it has employees in the covered area.  The Contractor is expected to make substantially uniform progress toward its goal in each craft during the period specified.

5.	Neither the provisions of any collective bargaining agreement, nor the failure by a union with whom the Contractor has a collective bargaining agreement, to refer either minorities or women shall excuse the Contractor's obligations under these specifications, Executive Order 11246, or the regulations promulgated pursuant thereto.

34

6.	In order for the nonworking training hours of apprentices and trainees to be counted in meeting the goals, such apprentices and trainees must be employed by the Contractor during the training period, and the Contractor must have made a commitment to employ the apprentices and trainees at the completion of their training, subject to the availability of employment opportunities.  Trainees must be trained pursuant to training programs approved by the U.S. Department of Labor.

7.	The Contractor shall take specific affirmative actions to ensure equal employment opportunity.  The evaluation of the Contractor's compliance with these specifications shall be based upon its effort to achieve maximum results from its actions.  The Contractor shall document these efforts fully, and shall implement affirmative action steps at least as extensive as the following:

a.	Ensure and maintain a working environment free of harassment, intimidation, and coercion at all sites, and in all facilities at which the Contractor's employees are assigned to work.  The Contractor, where possible, will assign two or more women to each construction project.  The Contractor shall specifically ensure that all foremen, superintendents, and other on-site supervisory personnel are aware of and carry out the Contractor's obligation to maintain such a working environment, with specific attention to minority or female individuals working at such sites or in such facilities.

b.	Establish and maintain a current list of minority and female recruitment sources, provide written notification to minority and female recruitment sources and to community organizations when the Contractor or its unions have employment opportunities available, and maintain a record of the organizations' responses.

c.	Maintain a current file of the names, addresses and telephone numbers of each minority and female off-the-street applicant and minority or female referral from a union, a recruitment source or community organization and of what action was taken with respect to each such individual.  If such individual was sent to the union hiring hall for referral and was not referred back to the Contractor by the union or, if referred, not employed by the Contractor, this shall be documented in the file with reason therefore, along with whatever additional actions the Contractor may have taken.

d.	Provide immediate written notification to the Director when the union or unions with which the Contractor has a collective bargaining agreement has not referred to the Contractor a minority person or woman sent by the Contractor, or when the Contractor has other information that the union referral process has impeded the Contractor's efforts to meet its obligations.

e.	Develop on-the-job training opportunities and/or participate in training programs for the area which expressly include minorities and women, including upgrading programs and apprenticeship and trainee programs relevant to the contractor's employment needs, especially those programs funded or approved by the Department of Labor.  The Contractor shall provide notice of these programs to the sources compiled under 7.b above.

f.	Disseminate the Contractor's EEO policy by providing notice of the policy to unions and training programs and requesting their cooperation in assisting the Contractor in meeting its EEO obligations; by including it in any policy manual and collective bargaining agreement; by publicizing it in the company newspaper, annual report, etc.; by specific review of the

35

policy with all management personnel and with all minority and female employees at least once a year and by posting the company EEO policy on bulletin boards accessible to all employees at each location where construction work is performed.

g.	Review, at least annually, the company's EEO Policy and affirmative action obligations under these specifications with all employees having any responsibility for hiring, assignment, layoff, termination or other employment decisions including specific review of these items with on-site supervisory personnel such as Superintendents, General Foremen, etc., prior to the initiation of construction work at any job site.  A written record shall be made and maintained identifying the time and place of these meetings, persons attending, subject matter discussed, and disposition of the subject matter.

h.	Disseminate the Contractor's EEO policy externally by including it in any advertising in the news media, specifically including minority and female news media, and providing written notification to and discussing the Contractor's EEO policy with other Contractors and Subcontractors with whom the Contractor does or anticipates doing business.

i.	Direct its recruitment efforts, both oral and written, to minority, female and community organizations, to schools with minority and female students and to minority and female recruitment and training organizations serving the Contractor's recruitment area and employment needs.  Not later than one month prior to the date for the acceptance of applications for apprenticeship or other training by any recruitment source, the Contractor shall send written notification to organizations such as the above, describing the openings, screening procedures, and tests to be used in the selection process.

j.	Encourage present minority and female employees to recruit other minority persons and women and, where reasonable, provide after school, summer and vacation employment to minority and female youth both on the site and in other areas of a Contractor's work force.

k.	Validate all tests and other selection requirements where there is an obligation to do so under 41 CFR Part 60-3.

l.	Conduct, at least annually, an inventory and evaluation at least of all minority and female personnel for promotional opportunities and encourage these employees to seek or to prepare for, through appropriate training, etc., such opportunities.

m.	Ensure that seniority practices, job classifications, work assignments and other personnel practices, do not have a discriminatory effect by continually monitoring all personnel and employment related activities to insure that the EEO policy and the Contractor's obligations under these specifications are being carried out.

n.	Ensure that all facilities and company activities are non-segregated except that separate or single-user toilet and necessary changing facilities shall be provided to assure privacy between the sexes.

o.	Document and maintain a record of all solicitations of offers for subcontracts from minority and female construction contractors and suppliers, including circulation of solicitations to minority and female contractor associations and other business associations.

p.	Conduct a review, at least annually, of all supervisors' adherence to and performance under the Contractor's EEO policies and affirmative action obligations.

36

8.	Contractors are encouraged to participate in voluntary associations which assist in fulfilling one or more of their affirmative action obligations (7.a through 7.p).  The efforts of a contractor association, joint contractor-union, contractor-community, or other similar group of which the contractor is a member and participant, may be asserted as fulfilling any one or more if its obligations under 7.a through 7.p of these specifications provided that the contractor actively participates in the group, makes every effort to assure that the group has a positive impact on the employment of minorities and women in the industry, ensures that the concrete benefits of the program are reflected in the Contractor's minority and female work force participation, makes a good faith effort to meet its individual goals and timetables, and can provide access to documentation which demonstrates the effectiveness of actions taken on behalf of the Contractor.  The obligation to comply, however, is the Contractor's and failure of such a group to fulfill an obligation shall not be a defense for the Contractor's non-compliance.

9.	A single goal for minorities and a separate single goal for women have been established.  The Contractor, however, is required to provide equal employment opportunity and to take affirmative action for all minority groups, both male and female, and all women, both minority and non-minority.  Consequently, the Contractor may be in violation of the Executive Order if a particular group is employed in a substantially disparate manner (for example, even though the Contractor has achieved its goals for women generally the Contractor may be in violation of the Executive Order if a specific minority group of women is underutilized).

10.	The Contractor shall not use the goals and timetables or affirmative action standards to discriminate against any person because of race, color, religion, sex, or national origin.

11.	The Contractor shall not enter into any Subcontract with any person or firm debarred from Government contracts pursuant to Executive Order 11246.

12.	The Contractor shall carry out such sanctions and penalties for violation of these specifications and of the Equal Opportunity Clause, including suspension, termination and cancellation of existing subcontracts as may be imposed or ordered pursuant to Executive Order 11246, as amended, and its implementing regulations, by the Office of Federal Contract Compliance Programs.  Any Contractor who fails to carry out such sanctions and penalties shall be in violation of these specifications and Executive Order 11246, as amended.

13.	The Contractor, in fulfilling its obligations under these specifications, shall implement specific affirmative action steps, at least as extensive as those standards prescribed in paragraph 7 of these specifications, so as to achieve maximum results from its efforts to ensure equal employment opportunity.  If the Contractor fails to comply with the requirements of the Executive Order, the implementing regulations, or these specifications, the Director shall proceed in accordance with 41 CFR 60-4.8.

14.	The Contractors shall designate a responsible official to monitor all employment related activity to ensure that the company EEO policy is being carried out to submit reports relating to the provisions hereof as may be required by the Government and to keep records.  Records shall at affiliation if any, employee identification number when assigned, social security number, race, sex, status (e.g., mechanic, apprentice, trainee, helper, or laborer), dates of changes in status, hours worked per week in the indicated trade, rate of pay, and locations at which the work was performed.  Records shall be maintained in an easily understandable and retrievable form; however, to the degree that existing records satisfy this requirement, contractors shall not be required to maintain separate records.

37

15.	Nothing herein provided shall be construed as a limitation upon the application of other laws which establish different standards of compliance or upon the application of requirements for the hiring of local or other area residents 	(e.g., those under the Public Works Employment Act of 1977 and the Community Development Block Grant Program).

16.	The Contractor will receive at the time of Award Federal Form CC-257 for his use in reporting monthly the Affirmative Actions for minority and female which he has employed.

38

APPENDIX A

The following goals and timetables for female utilization shall be included in all Federal and federally assisted construction contracts and subcontracts in excess of $10,000.  The goals are applicable to the Contractor's aggregate on-site construction work force whether or not part of that work force is performing on a Federal or federally assisted construction contract or subcontract.

AREA COVERED:  Nationwide

GOALS AND TIMETABLES

Timetable								        Goals

(percent)

From April 1, 1978 until March 31, 1979...........................................................	3.1

From April 1, 1979 until March 31, 1980...........................................................	5.0

From April 1, 1980 until further notice...............................................................	6.9

39

APPENDIX B

Until further notice, the following goals for minority utilization in each construction craft and trade shall be included in all Federal or federally assisted construction contracts and subcontracts in excess of $10,000 to be performed in the respective geographical areas.  The goals are applicable to each nonexempt contractor's total on-site construction work force, regardless of whether or not part of that work force is performing work on a Federal, federally assisted or non-federally related project, contract or subcontract.

Construction contractors which are participating in an approved Hometown Plan (see 41 CFR 60-4.5) are required to comply with the goals of the Hometown Plan with regard to construction work they perform in the area covered by the Hometown Plan.  With regard to all their other covered construction work such contractors are required to comply with the applicable SMSA or EA goal contained in this appendix B-80.

Goal

State	(percent)

Maryland:

019 Baltimore, MD:

SMSA Counties:

0720 Baltimore, MD.................................................................................................	23.0

MD Anne Arundel; MD Baltimore;

MD Carroll; MD Harford;

MD Howard; MD Baltimore City

Non-SMSA Counties.....................................................................................................	23.6

MD Caroline; MD Dorchester;

MD Kent; MD Queen Anne’s;

MD Somerset; MD Talbot;

MD Wicomico; MD Worchester

Washington, DC:

020 Washington, DC:

SMSA Counties:

8840 Washington, DC.................................................................................................	28.0

MD Charles; MD Montgomery;

MD Prince Georges

Non-SMSA Counties...................................................................................................	25.2

MD Calvert; MD Frederick

MD St. Mary’s; MD Washington

Pennsylvania

Non-SMSA Counties.....................................................................................................	 4.8

MD Allegany: MD Garrett

40

TRAINING PROVISIONS

As part of the Contract's Equal Employment Opportunity Affirmative Action Program, on-the-job training shall be provided as follows:

The on-the-job training shall be aimed at developing full journeypersons in the type of trade or job classification involved.  On this Contract ZERO (0) (number to be filled in by the Administration) persons will be trained.

In the event that a Contractor subcontracts a portion of the Contract work, the Contractor shall determine how many, if any, of the trainees are to be trained by the subcontractor, however, the Contractor shall retain the primary responsibility for meeting the training requirements imposed by this Provision.  The Contractor shall also insure that this training Provision is physically included in each subcontract to insure that the workforce utilized by the subcontractor meet the goals for minority and female employment and training.  Where feasible, 25 percent of apprentices or trainees in each occupation shall be in their first year of apprenticeship or training.

The number of trainees in each classification shall be distributed among the work classifications on the basis of the Contractor's needs, minority and women employment goals specified for each trade in the Contract Provision, and the reasonable area of recruitment.

Prior to beginning construction, the Contractor shall submit to the Administration for approval Manpower and Training Utilization (MTU) Schedule no later than at the preconstruction meeting.

The MTU schedule shall include:

1.	The proposed training programs.

2.    The number of trainees to be trained in each classification.

3.	Anticipated starting and ending dates for training in each classification.

No Contract work may be undertaken until the Administration has accepted the schedule.

If the submitted training programs fail to meet the requirements as defined within these Provisions, the Administration will withhold one percent of the total category code one pay items from the payment due the Contractor.  The Contractor shall submit a revised Manpower and Training Utilization Schedule when major changes in the Contract work schedule occur that substantially affect the previously submitted schedule.

The Contractor shall be credited for each trainee employee who is currently enrolled or becomes enrolled in an approved program and will be reimbursed for the hourly cost of the trainee as specified in the schedule of prices.

Training and upgrading of minorities and women toward journeyperson status is a primary objective of this Training Provision.  The purpose for this objective is to insure a pool of qualified minorities and women to replace those journeypersons who, in the natural course of events will leave the workforce.   The program will also provide opportunities to the minorities and women trainees in geographic areas where shortages in minority and women journeypersons are prevalent and recognized due to the Contractor's inability to meet the Equal Employment Opportunity goals specified in this Contract.

41

The training requirements of this Training Provision are not intended nor shall they be used to discriminate against any applicant for training, whether a member of a protected class or not.  It is the Contractor's responsibility to demonstrate good faith efforts to ensure an adequate workforce representation of minorities and women in all job classifications on this Contract.  Therefore, the Contractor shall consider the employment Contract goals set for minorities and females when enrolling trainees.  The Contractor's

utilization of the on-the-job training goals will be weighed when an Equal Employment Opportunity workforce compliance determination is made.

The Contractor shall make every effort to enroll minority and women trainees (e.g., by conducting systematic and direct recruitment through public and private sources likely to yield minorities and women to the extent that these persons are available within a reasonable area of recruitment).

No employee shall be employed as a trainee in any classification which the individual has successfully completed a training program leading to journeyperson status or has been employed as a journeyperson.  This includes a person gainfully employed as a journeyperson by virtue of informal on-the-job training.  The Contractor should satisfy this requirement by including appropriate questions in the employee job application or by other suitable means.  Regardless of the method used, the Contractor's records shall document the findings in each case.  In the case of apprentices, evidence of indentureship and registration of the approved apprenticeship program shall be included in the Contractor's records.

The minimum length and type of training and rate for each classification shall be specified in the training program by the Contractor and approved by the Administration and the Federal Highway Administration.

The Administration will approve any program specified in the Administration's On-The-Job Training Manual.  The Administration and the Federal Highway Administration will consider other programs if it is reasonably calculated that the programs conform to the Equal Employment Opportunity obligations of the Contract and will qualify the average trainee for journeyperson status in the specified classification by the end of the training period.  Apprenticeship programs registered with the U.S. Department of Labor, Bureau of Apprenticeship and Training, or with a State apprenticeship agency recognized by the Bureau, and training programs approved by, but not necessarily sponsored by the U.S. Department of Labor, Employment and Training Administration, Bureau of Apprenticeship and Training will also be acceptable, provided that the program being offered is administered in a manner consistent with the Equal Employment obligation of Federal-aid highway construction Contracts and meets the minimum requirements of this Training Provision.

Some offsite training is permissible as long as the training is an integral part of an approved training program and does not comprise a significant part of the overall training.

Unless otherwise specified, the Contractor will be reimbursed 80 cents per hour of training given an employee on this Contract in conformance with an approved training program.  As approved by the Project Manager, reimbursement will be made for training persons in excess of the number specified herein. This reimbursement will be made even though the Contractor received additional training program funds from other sources, provided that the other sources do not specifically prohibit the Contractor from receiving other reimbursement.  Reimbursement for offsite training indicated above will only be made to the Contractor where the Contractor does one or more of the following and the trainees are concurrently employed on a Federal-aid project:

1.	Contributes to the cost of the training.

2.	Provides the instruction to the trainee or pays the trainee's wages during the offsite training period.

42

No payment will be made to the Contractor if either the failure to provide the required training or the failure to hire the trainee as a journeyperson is caused by the Contractor and evidences a lack of "good faith" on the part of the Contractor in meeting the requirements of this Training Provision.  It is normally expected that a trainee will begin training on the project as soon as feasible after the start of work utilizing the skill involved and remain on the project as long as training opportunities exist in the work classification or until the program is completed.  It is not required that all trainees be on board for the entire length of the Contract.  A Contractor will have fulfilled their responsibilities under this Training Provision when:

1.	Systematic and direct recruitment likely to yield qualified minority and women applicants is conducted through:

a.	Public and private referral sources.

b.	Advising the existing workforce of training opportunities.

c.	Unions (if applicable).

2.	Acceptable training has been provided to trainees enrolled in the program.

3.	The number of specified trainees have completed the minimum hours required in an approved training program.

4.	Trainees completing approved programs are retained in the workforce as journeypersons.

The Contractor shall pay the trainees at least 60 percent of the appropriate minimum journeyperson's hourly rate plus the full fringe benefits specified in the Contract for the first half of the training period, 75 percent for the third quarter of the training period plus full fringe benefits, and 90 percent for the last quarter of the training period plus full fringe benefits.  However, in no case shall the total hourly rate be less than the U.S. Department of Labor's unskilled laborer wage rate for the project.  In addition, all trainees shall be identified as such on the certified payroll.  The Contractor shall furnish the trainee a copy of the approved training program in which the trainee is enrolled.  The Contractor shall provide each trainee with a certificate showing the type and length of training satisfactorily completed.  The Contractor shall submit a Certificate to the trainee in the following instances:

1.	Certificate of Completion when a trainee completes the total number of hours required to complete a training program.

2.	Certificate of Training when a trainee does not totally complete the required program hours.

The Contractor shall provide for the maintenance of records and furnish periodic reports inclusive of the Administration's Contractor's Semiannual Training Reports, documenting his performance under this Training Provision.  The Semiannual Training Report is to be submitted by the 10th of the month following the reporting period (July 10 and January 10).If the Contractor fails to fully comply with these Training Provisions, the Administration's Representative will make a final report of non-compliance to the Administrator, who may direct the imposition of one or both of the sanctions listed below:

1.	Withholding a percentage of the progress payment.

2.	Other action appropriate and/or within the discretion of the Administrator.

43

NOTICE TO ALL HOLDERS OF THIS CONTRACT DOCUMENT

HIGH VISIBILITY SAFETY APPAREL POLICY

BACKGROUND

Research indicates that high visibility garments have a significant impact on the safety of employees who work on highways and rights-of-way.  In addition, high visibility garments may help to prevent injuries and accidents and to make highway workers more visible to the motoring public, which ultimately improves traffic safety.

STATEMENT OF POLICY

(a)	The High Visibility Safety Apparel Policy provides a standardized apparel program.

(b)	The program seeks to improve the visibility of all persons who work on Administration highways and rights-of-way.

(c)	All apparel shall contain the appropriate class identification label.

(d)	Compliance with this policy is retroactive and becomes effective immediately.  All affected employees shall receive high visibility apparel awareness training.

APPLICABILITY

This policy applies to all Administration employees and all other persons who work on Administration highways and rights-of-way.  All workers shall wear, at a minimum, Class 2 ANSI/ISEA 107/2004 apparel.

(a)	For Administration employees, this apparel shall have a fluorescent yellow-green background material color and be the outermost garment worn.

(b)	Retro-reflective material color for Administration employee apparel shall be silver or white and be visible at a minimum distance of 1,000 feet.  The retro-reflective safety apparel shall be designed to clearly recognize and differentiate the wearer from the surrounding work environment.  The retro-reflective material may be contrasted by fluorescent orange background material not exceeding one and one half inches on either side of the retro-reflective material.

(c)	For non-Administration employees, this apparel shall be either fluorescent orange-red or fluorescent yellow-green background material color and be the outermost garment worn.

(d)	Retro-reflective material color for non-Administration employee apparel shall either be orange, yellow, white, silver, yellow-green, or a fluorescent version of these colors, and be visible at a minimum distance of 1,000 feet.  The retro-reflective safety apparel shall be designed to clearly recognize and differentiate the wearer from the surrounding work environment.

44

REFERENCES

(a)	ANSI/ISEA 107/2004 standard – American National Safety Institute/International Safety Equipment Association

(b)	MUTCD 2003 – Manual for Uniform Traffic Control Devices - Sections 6D.03B and 6E.02

(c)	Visibility Research – The VCTR 1989 report concludes that fluorescent colors, when compared with non-fluorescent colors, enhance the daytime conspicuity of worker clothing.

DEFINITIONS

(a)	Apparel – The outermost high-visibility garment worn by employees who work on Administration highways and rights-of-way.

(b)	Highways – All roads owned by the Maryland Department of Transportation and maintained by the Administration.

(c)	High Visibility – The ability for workers to be distinguishable as human forms to be seen, day and night, at distances that allow equipment operators and motorists to see, recognize, and respond.

45

PROJECT DESCRIPTION

(This method and format to be used on all projects.  The Project Engineer is responsible for this information)

(Example)This project, located in the City of Aberdeen is for the construction of sidewalk on Mt. Royal Avenue from the intersection of West Bel Air Avenue to Williams Street.

The work will consist of the following:

- Construction of 1350 feet of sidewalk on Mt. Royal Avenue

- Construction of 14 driveway entrances

- Construction of 7 handicap ramps

- Installation of 4 crosswalks

- Installation of 4 storm drains

- Installation of 4 school zone signs.

(THE ENGINEER WILL DESCRIBE THE MAJOR WORK INCLUDED IN THE PROJECT.)

(Use as much space as is necessary)

(For Projects without Plans, if necessary, use this space to list miscellaneous items of work, i.e. )

INLETS.

15 foot COG Station 15+50 Left

CULVERTS.

50 LF 24 in. RCCP Station 15+50

SPECIFICATIONS

All work on this project shall conform to the Maryland Department of Transportation, State Highway Administration's Specifications entitled, "Standard Specifications for Construction and Materials" dated July 2008 revisions thereof, or additions thereto, and the Special Provisions included in this Invitation for Bids.

46

NOTICE TO CONTRACTOR

PROJECT SCHEDULE.  Section 109 shall only apply when a CPM Project Schedule item is included in the Schedule of Prices.  Otherwise, all Project Schedules shall conform to Section 110.

NOTICE TO BIDDERS.  The Proposal Form Packet in this Invitation for Bids requires the following information be submitted for the Bidder and each firm quoting or considered as subcontractors:

(a)	Name of firm.

(b)	Address of firm.

(c)	MBE, Non-MBE, DBE, or Non-DBE.

(d)	Age of firm.

(e)	Annual gross receipts per last calendar year.

Note that there are provisions for submitting copies for additional subcontractors, and that an “X” is required to indicate whether or not additional copies have been submitted.

AFFIRMATIVE ACTION PLAN (AAP) CONTRACT GOALS.  In order to be in compliance with the revised MBE/DBE laws effective November 19, 2008 the bidder is required to complete the AAP information on pages 17, 18, 21, 22, 23, and 24 of 28 of the Contract Provisions, Proposal Form Packet —Federal, or complete the AAP information on pages 16, 17, 20, 21, 22, and 23 of 28 of the Contract Provisions, Proposal Form Packet—State, or complete the AAP information on pages  17, 18, 21, 22, 23, and 24 of 29 of the Contract Provisions, Proposal Form Packet—State Small Business Reserve Procurement.  Failure to complete the information may be grounds for the bid to be declared non-responsive.

BOOK OF STANDARDS.  The Book of Standards for Highway and Incidental Structures is now available only on the Administration’s Internet Site at www.marylandroads.com.  The Book of Standards can be located by clicking on Business with SHA; Business Standards and Specifications; and Book of Standards for Highway and Incidental Structures.  Hard copies of the Book of Standards will no longer be sold in the Cashiers Office and hard copy distributions of the Standard updates will no longer be made.

47

REQUEST FOR INFORMATION.  Any information regarding the requirements or the interpretation of any provision of the Contract Documents shall be requested, in writing, and delivered prior to the scheduled date of bid opening.  Responses to questions or inquiries having any material effect on the bids shall be made by written addenda sent to all prospective bidders. The Administration will not respond to telephone requests for information concerning this invitation for bids that would materially affect the bid.

Written requests for information or questions shall be addressed to:

(Example)

City of Aberdeen 					Project Manager: 	Phyllis Grover

60 North Parke Street

Aberdeen, MD 21001

410-272-1600 ext. 216

410-273-7402 (fax)

Each request for information or questions shall include the Contract number and the name and address of the originator.

(Use this area to alert the bidders to special out of the ordinary requirements pertaining to the project such as incentive/disincentive requirements, limited access to the project site, or whenever additional restraints, restrictions, limits, etc. are being included on a project.  This is also a good place to alert bidders of major Specification changes when they are initiated, but should be later removed from the Notice to Contractor after they have been used for a few projects and become more of a standard operating procedure or an Special Provisions Insert.

The Right-of-Way Status and the Permit Status should be included under Notice to Contractor only when there are minimum statements such as only a few right-of-ways are yet to be obtained or that no permits are required for the Project.  However, delete them from the Notice to Contractor and place them on separate pages with all backup information whenever a bulk of information is to be included.)

RIGHT-OF-WAY STATUS

All work will take place within the City of Aberdeen right-of-way.

LICENSES AND CERTIFICATIONS

Within 10 days of written notice to proceed, the Contractor shall submit current copies of all required Certificates, Licenses, and Insurances to the Project Manager:

- Maryland Business License

- Insurance Certificate(s)

TRAFFIC CONTROLS AND WORK SCHEDULE

Prior to the commencement of work, the Contractor shall submit the following to the Project Manager for review and approval:

- Traffic Control Plan(s)

- Work Schedule and Plan

48

NOTICE TO CONTRACTOR

EARLY SUBMISSIONS.  The last sentence of the first paragraph of TC-5.02, “No work shall be started before written receipt of the Notice to Proceed” shall not apply to the following:

After notification to the Contractor from the Administration that the Contractor is the apparent low bidder, the Contractor will be permitted to provide a written request to the Engineer to submit documentation for materials sources and working drawings for any items of work that have a long lead time and could jeopardize the project schedule. Upon written approval from the Engineer the Contractor may submit the applicable documentation to the Engineer.

Should the Contract not be awarded to the apparent low bidder who meets the requirements of the Contract, GP-8.10 will apply for all costs accrued for the preparation and approval of the working drawings and any resultant material purchase approved by the District Engineer and steel fabricated in conformance with the approved working drawings between the date the Contractor received notice of apparent low bidder and the date of notice that the apparent low bidder will not be awarded this Contract.

Should this Contract not be awarded to the apparent low bidder due to failure of the Contractor to comply with all award and execution requirements, all costs accrued for the preparation of the specific items and any resultant material purchased and steel fabrication shall be borne by the Contractor.

Failure of the Contractor to submit the early submissions will not be basis for delaying issuance of the Notice to Proceed or be considered a reason for a time extension.

49

# Invitation to Bid

(Example)This Invitation to Bid is for of the construction of 1350+ feet of sidewalks, installation of handicap ramps, and replacement of 14 driveway entrances/aprons on Mt. Royal Avenue from the intersection of West Bel Air Avenue and Mt. Royal Avenue to Williams Street.

Issuing Office

City of Aberdeen, Department of Planning and Community Development

60 N. Parke Street, Aberdeen, Maryland 21001

Attn: Phyllis Grover

Office:    410-272-1600 ext. 216

Request for Proposal Schedule and Submission

A pre-bid meeting will be held on November 4, 2013 at 3:00 p.m. at Mt. Royal Avenue in front of the sidewalk near Aberdeen Middle School (on site) , Aberdeen MD 21001.

Written or faxed requests for clarification and additional information must be received in the Department of Planning and Community Development not later than November 8, 2013 to prepare and circulate any necessary addenda to all offerors.

One (1) original and one (1) sealed copies of the proposal must be received in the Department of Planning and Community Development no later than 3:00 p.m. on November 18, 2013.  There will be no public opening. The proposal must be accompanied by a brief transmittal letter, signed by an officer authorized to bind the firm to its proposal, with required affidavit(s) attached.  Proposals submitted in response to this RFP are irrevocable for 60 days after the proposal due date.  Award of the contract is anticipated within 15 days of closing date.

A bid bond or a certified check in the amount of five (5%) percent of the bid amount is required with the submission of a bid.

As soon as the bid prices have been compared, the City will return the bonds of all except the three (3) lowest responsible Bidders.

The bonds of the two remaining lowest responsible Bidders will be retained until the performance/payment surety has been provided and approved and the Contract Agreement executed, after which the bid bonds will be returned.

50

All bids shall be submitted (typed or written in ink) on the attached Bid Form.  Bids made on any other than the attached Bid Form will not be considered.

The contractor must acknowledge receipt of all addenda, if any, on the Bid Form.

The bid will be awarded to the LOWEST RESPONSIBLE BIDDER.

Reservations

The City reserves the right to cancel this RFP at any time after issuance, to reject, in whole or in part, any and all offers received, to waive minor technicalities in proposals, and to negotiate with responsible offers in any manner necessary to serve its best interests.

Addenda

Any necessary additions or corrections to this RFP will be made by addenda, and issued to all offerors of record. Addenda become part of the RFP, and must be acknowledged by each offeror; failure to acknowledge any addenda shall not relieve offerors of compliance with the terms thereof. The City assumes no responsibility for oral instructions.

Wage Rates

All laborers and mechanics employed by the contractor and any subcontractors shall be paid wages at rates not less than those prevailing on similar projects in the locality as determined by the U.S. Department of Labor under the Davis-Bacon and related Acts.

Economy of Preparation

Proposals should be prepared simply and economically, providing a straightforward, concise description of offeror ability to satisfy the requirements of this RFP.

Incurred Expenses

Offerors are responsible for proposal preparation and submission costs, as well as travel costs incurred in connection with oral presentations or other pre-award discussions.

Acceptance of Terms and Conditions

By submitting a proposal in response to this RFP, the offeror accepts the terms and conditions set forth herein.

Public Information Act Notice

Offerors shall identify any portions of their proposals deemed to contain confidential or proprietary information or trade secrets, and provide justification of why such material, upon request, should not be disclosed.

51

Evidence of Offeror Responsibility

The City may require offerors to submit additional information regarding technical expertise, and other qualifications, and may consider any information otherwise available concerning those qualifications. The City may make such investigation as it deems necessary to determine offeror responsibility.

Award without Discussions

The City reserves the right to accept the best written proposal without further discussions, and may do so; thus, offeror should ensure that the initial proposal is both complete, and competitively priced.

Contractor Responsibilities; Subcontractors

The City will enter into a contract with the selected offeror only, and that offeror shall be responsible for all products and services required by the RFP. Subcontractors, if any, must be identified in the proposal, with a complete description of their role relative to the offeror.

Conflicts of Interest

The offeror shall identify any actual or potential conflicts of interest that exist, or which may arise if the offeror is recommended for award, and propose how such conflicts might be resolved.

Financial Disclosure

The successful offeror shall comply with the provisions of the Annotated Code of Maryland, State Finance and Procurement Article §13-221 which requires that every business that enters into contracts, leases or other agreements with the State of Maryland or its units or both, under which the business is to receive a total of $100,000 or more shall, within 30 days of the time when the total value of these contracts, leases or other agreements reaches $100,000, file with the Secretary of State a list that contains the name and address of: (1) any resident agent of the business; (2) each officer of the business; and (3) if known, each person who has beneficial ownership of the business.

Political Contribution Disclosure

The successful offeror shall comply with the provisions of Article 33, §30-1 et seq. of the Annotated Code of Maryland, which requires that every bidder or contractor doing more than $100,000 or more of business with the State, a County, incorporated municipality or other political subdivision are required to file periodic reports of political contributions in excess of $500 to candidates for elective office in the State. Contact the Division of Candidacy and Campaign Finance, 410-974-3711, ext. 5 or 800-222-8683, ext. 5 for forms and further information.

52

Anti-Bribery Affidavit

Section 16-202, State Finance and Procurement Article requires that each offeror seeking a contract submit an affidavit stating whether the entity or any of its officers, directors, or partners, or any of its employees directly involved in obtaining contracts with the State, have been convicted of bribery, attempted bribery, or conspiracy to bribe under the laws of any state or the federal government. The affidavit form that must be returned with each proposal is included with this RFP.

Non-collusion

By its signature on the proposal documents submitted, the successful offeror attests that its agents, servants and/or employees, to the best of its knowledge and belief, have not in any way colluded with anyone for and on behalf of the offeror, or themselves, to obtain information that would give the offeror an unfair advantage over others, nor has it colluded with anyone for and on behalf of the offeror, or itself, to gain any favoritism in the award of this RFP.

Compliance with Laws

By submitting a response to this solicitation, offeror represents that it is not in arrears in the payment of any obligation due and owing the City of Aberdeen, Harford County or the State of Maryland, including the payment of taxes and employee benefits, and that it shall not become so in arrears during the term of any contract arising from award of this RFP.

Governing Law

The laws of the City of Aberdeen, Harford County, and the State of Maryland, and where applicable, federal law and regulation, will govern the contract awarded pursuant to this RFP.

Ownership and Retention of Records

All reports, drawings, and other data prepared under the contract issued pursuant to this RFP shall become the property of the City. Unless otherwise required by applicable statute of limitations, the successful offeror shall retain all records and documents related to any contract awarded pursuant to this RFP for 3 years after final contract payment by the City, and shall make them available for inspection and audit by authorized representatives of the City at all reasonable times.

Billing and Payment

The successful offeror shall keep accurate, document records of time, material, and transportation allocable to the City’s contract. Related records will be available for audit purposes during normal business hours, as often as deemed necessary. Invoices submitted for payment must be reviewed by the City Project Manager.  Requests for payment must be submitted to:

City of Aberdeen, Department of Planning and Community Development

Attention:  Phyllis Grover, Director of Planning and Community Development

60 N. Parke Street

Aberdeen, MD  21001

53

Applicable Standards/Guidelines

The Contractor is required to perform all services in accordance with generally accepted standards of professional practice.

City Project Manager

The City of Aberdeen Director for the Department of Planning and Community Development will manage and coordinate this project.  All communications related to this project shall be directed to:

Phyllis Grover, Director of Planning and Community Development

City of Aberdeen

60 N. Parke Street

Aberdeen, MD 21001

410-272-1600 ext. 216

Insurance Requirements

Prior to the execution of the contract, the successful offeror must obtain, at its own cost and expense, and keep in full force and effect until termination of the contract, the following insurance, written in companies licensed to do business in the State of Maryland.

The coverage will be evidenced by a certificate of insurance issued directly to the City by the offeror’s agent, and provide 60 days’ written notice to the City of cancellation or material change in coverage. A two-year extended reporting provision is required to safeguard against gaps in coverage after policies are terminated. All liability policies shall name the City of Aberdeen, Maryland as an additional insured.

Required Coverage and Limits:

1. Architects and Engineers errors and omissions insurance (professional liability):	$1,000,000

2. Valuable Papers Insurance:	  35% of fee

3. Automobile Liability (owned, hired and non-owned automobiles): 	$1,000,000

Bodily injury, person		$1,000,000

Bodily injury, per occurrence 	  $1,000,000

Property damage, per occurrence	  $1,000,000

4. Commercial General Liability:	$1,000,000

Bodily injury, property damage or medical expenses, per occurrence:	$1,000,000

Bodily injury, property damage and personal injury claims:	$1,000,000

5. Workers Compensation:		Statutory limit

54

Evaluation and Selection Criteria

Responsiveness: The City of Aberdeen staff will initially evaluate each Bid from the information at hand against the criteria established herein to determine if the Bid is responsive according to the solicitation requirements. Questions of a clarifying nature may be asked from bidders as required. Bids lacking any of the submission criteria may be determined non-responsive and may be removed from consideration.

The lowest responsible Bidder shall be the basis of award.  If the bidder to whom an award is made shall fail to execute the contract within fifteen (15) calendar days from notice-of-award, the award may be annulled and the contract awarded to the second lowest responsible Bidder and such Bidder shall fulfill every stipulation embraced herein as if he were the original party to whom the award was made or the City of Aberdeen may reject any or all bids as its interest may require.

Description of the Project

- Construction of 1350+ feet of sidewalk, installation of handicap ramps, and replacement of 14 driveway aprons on Mt. Royal Avenue from the intersection of West Bel Air Avenue to Williams Street.

- Remove and replace various types of curb at locations given or as directed by the City Engineer.

- Backfill, seed and mulch behind each replaced curb.

- Backfill, seed and mulch behind each replaced ramp.

For type of Material please see special provisions inserts.

- Install sidewalks as per Plates outlined in “Attachment A”

- Daily Cleanup.  The Contractor shall perform daily clean-up of all excess materials and removal all trash, materials, equipment, etc.

Performance Surety

A performance / payment surety in the amount of 100% of the contract price is required within fifteen (15) calendar days from notice-of-award.

Maintenance Surety

The Contractor further agrees to provide to the Owner a Maintenance Guaranty Surety guaranteeing all work for a period of one (1) year from the date of completion of the work and approved by the Owner.  The Maintenance Surety shall be 25% of the amount of the final contract cost and shall be furnished by the surety which furnished the Performance and Payment Sureties.   The Maintenance Surety shall be furnished prior to the release of final retainage payment to the Contractor.

55

CATEGORY 900

MATERIALS

665	DELETE:	SECTION 902 — PORTLAND CEMENT CONCRETE AND RELATED PRODUCTS in its entirety.

INSERT:	The following.

SECTION 902 — PORTLAND CEMENT CONCRETE AND RELATED PRODUCTS

902.01 STORAGE.  Storage of materials shall conform to the Contract Documents and as directed by the Engineer.

902.02 CERTIFICATION OF PORTLAND CEMENT AND BLENDED HYDRAULIC CEMENT.  The manufacturer shall furnish certification as specified in TC-1.02.  The certification shall also include:

(a)	The mill shall report its quality control procedures, and submit a new report whenever there is a procedural change.

(b)	The mill's control laboratory shall be inspected by the Cement and Concrete Reference Laboratory of the National Institute of Standards and Technology on their regularly scheduled visits.  The Engineer shall be provided with copies of the reports of these inspections along with an account of the action taken to correct cited deficiencies.

(c)	Records of data accumulated by the quality control procedures shall be produced upon request.

(d)	A certified document shall accompany each shipment stating that the contents conform to all applicable requirements.  Additionally, the document shall show the producer's name, mill location, carrier number, date loaded, weight contained in carrier, silo number, consignee, destination, Contract number, and type of cement.  The signature and title of the signer shall be shown on the document.

(e)	The mill shall, upon request, supply certified chemical and physical test values that can be associated with any sample representing cement drawn from a particular silo on a given date.

(f)	Acceptance of cement by certification will be terminated if test results differ from mill results by more than the precision limits given in the test method.  The acceptance procedure will then revert to storage testing and approval prior to shipment.

902.03 HYDRAULIC CEMENT.

902.03.01 Portland Cement.  M 85, with the fineness and the time of setting determined using T 153 and T 131, respectively.

56

902.03.02 Ground Iron Blast Furnace Slag.  M 302, Grade 100 or 120.  The Contractor may request to substitute a maximum of 50 percent of the weight of cement with ground iron blast furnace slag.  When ground iron blast furnace slag is used, the minimum cement factor and water/cement ratio will be determined on the basis of the combined weight of the portland cement and ground iron blast furnace slag.  When ground iron blast furnace slag is used to control alkali silica reactivity, see Table 902 B for percentage.

902.04 BLENDED HYDRAULIC CEMENT.  M 240, Type I (PM) or a Type IP containing 15 to 25 percent pozzolan by weight of cement.  Maximum loss on ignition is 3.0 percent. Do not use ground iron blast furnace slag for blending.  The requirement for a manufacturer’s written statement of the chemical composition is waived.

902.05 MASONRY CEMENT.  C 91, except the water retention and staining tests are waived.

902.06 CONCRETE ADMIXTURES.  Do not use concrete admixtures that contribute more than 200 ppm of chlorides based on the cement content when tested per MSMT 610.  Use only prequalified admixtures.

Do not use pozzolan and Type I (PM) or Type IP cement in the same mix.  Since the strength gains are delayed with these materials, a longer period of time may be required for curing and form removal.

902.06.01 Air Entraining Admixtures.  M 154.

902.06.02 Chemical Admixtures.  M 194, Type A, D, or nonchloride C.

902.06.03 High Range Water Reducing Admixtures.  M 194, except that it shall be a liquid, the water content shall be a maximum of 85 percent of that of the control, and the durability factor shall be a minimum of 90.  Use Type F for early strength, which shall produce a minimum compressive strength in 12 hours of 180 percent of that of the control.  Use Type G when early strength is not specified.  The manufacturer shall furnish certification as specified in TC-1.02.  The certification shall include curves indicating the fluid ounces of admixture per 100 lb of cement as related to water reduction and strength gain for 12 hours when used with a minimum cement factor of 700 lb.

902.06.04 Pozzolans.  The use of pozzolans may be requested to control alkali silica reactivity or for other reasons.  When a pozzolan is used, determine the minimum cement factor and water/cement ratio on the basis of the combined weight cement and pozzolan.  See Table 902 B for percentage of fly ash, and microsilica.

(a)	Fly Ash.  M 295, pozzolan Class C or F, except that the maximum permissible moisture content shall be 1.0 percent, and when used in concrete Mix Nos. 3 and 6 the maximum loss on ignition 3.0 percent.

(b)	Microsilica.  C 1240, except that the oversize requirement is waived.

57

902.06.05 Corrosion Inhibitors.  Corrosion inhibitors shall be calcium nitrite based and contain a minimum of 30 percent active ingredients by mass.  The gallonage of corrosion inhibitor used in the concrete mixture shall be included as water when determining the water/cementitious materials ratio.

902.07 PORTLAND CEMENT CONCRETE CURING MATERIALS.  Use burlap cloth, sheet materials, liquid membrane forming compounds, or cotton mats.

902.07.01 Burlap.  M 182, Class 1, 2, or 3.

902.07.02 Sheet Materials.  M 171 with the following exceptions:

(a)	White Opaque Burlap Polyethylene Sheeting.  Tensile strength and elongation requirements are waived. Use sheeting having a finished product weight of not less than 10 oz/yd2.

(b)	White Opaque Polyethylene Backed Nonwoven Fabric.  902.07.02(a), with the thickness requirement waived.  Use material having a finished product weight of not less than 5 oz/yd2.

(c)	White Opaque Polyethylene Film.  Tensile strength and elongation requirements are waived.

902.07.03 Liquid Membrane.  M 148. Field control testing of the white pigmented curing compounds is on the basis of weight per gallon.  The samples shall not deviate more than  0.3 lb/gal from the original source sample.

902.07.04 Cotton Mats.  Cotton mats consist of a filling material of cotton bats or bats covered with unsized cloth and tufted or stitched to maintain the shape and stability of the unit under job conditions of handling.

Use coverings of either cotton cloth, burlap or jute having the following properties:

(a)	Cotton cloth covering shall weigh not less than 6.0 oz/yd2 and have an average of not less than 32 threads/in. of warp and not less than 28 threads/in. of filling.  Use raw cotton, cotton comber waste, cotton card strip waste, or combinations thereof as the raw material used in the manufacture of the cotton cloth.

(b)	Burlap or jute covering for cotton mats shall weigh not less than 6.4 oz/yd2 and shall have not less than of 8 threads/in. of warp and not less than 8 threads/in. of filling.  Use the grade known commercially as "firsts" and they shall be free from avoidable imperfections in manufacture and from defects or blemishes affecting the serviceability.

Use a cotton bat, or bats made of raw cotton, cotton waste, cotton linters, or combinations thereof, as the filling material for the mats. Mats shall weigh not less than 12 oz/yd2.

58

902.08 FORM RELEASE COMPOUNDS.  Use form release compounds that effectively prevent the bond of the concrete to the forms.  Form release compounds shall not cause discoloration of the concrete or adversely affect the quality or rate of hardening at the interface of the forms.

The flash point of the form release compound shall not be less than 100 F when tested per T 73.

902.09 PARAFFIN WAX.  Use clear paraffin wax for use as a bond breaker for concrete.  The flash point shall not be less than 380 F when tested under D 92.

902.10 PORTLAND CEMENT CONCRETE.  Section 915 and as specified herein.

902.10.01 Proportioning.  Prior to the start of construction, submit to the AME the source and proportions of materials to be used for each concrete mix.  The mixture shall meet 902.10.03.

The concrete, with the exception of water and chemical admixtures, shall be proportioned by weight.  Water and chemical admixtures may be proportioned by volume or weight.  The mix shall be uniform and workable.

902.10.02 Materials.

Coarse Aggregate 	901.01

Fine Aggregate	901.01

Cement	902.03 and 902.04

Concrete Admixtures	902.06

Synthetic Fibers	902.15

Water	921.01

902.10.03 Portland Cement Concrete Mixtures.

59

The concrete mixes shall conform to the following:

TABLE 902 A

| PORTLAND CEMENT CONCRETE MIXTURES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MIX NO. | 28 DAY SPECIFIED COMPRESSIVE STRENGTH psi | STANDARD DEVIATION psi | CRITICAL VALUE psi | MIN CEMENT FACTOR lb/yd3 | COARSE AGGREGATE SIZE M 43 / M 195 | MAX WATER/ CEMENT RATIO by wt | SLUMP RANGE in. | TOTAL AIR CONTENT % | CONCRETE TEMPERATURE F |
| 1 | 2500 | 375 | 2430 | 455 | 57, 67 | 0.55 | 2 — 5 | 5 — 8 | 70 ± 20 |
| 2 | 3000 | 450 | 3010 | 530 | 57, 67 | 0.50 | 2 — 5 | 5 — 8 | 70 ± 20 |
| 3 | 3500 | 525 | 3600 | 580 | 57, 67 | 0.50 | 2 — 5 | 5 — 8 | 70 ± 20 |
| 4 | 3500 | 525 | 3600 | 615 | 57, 67 | 0.55 | 4 — 8 | N/A | 70 ± 20 |
| 5 | 3500 | 525 | 3600 | 580 | 7 | 0.50 | 2 — 5 | 5 — 8 | 70 ± 20 |
| 6 | 4500 | 675 | 4770 | 615 | 57, 67 | 0.45 | 2 — 5 | 5 — 8 | 65 ± 15 |
| 7 | 4200 | 630 | 4420 | 580 | 57 | 0.50 | 1½ — 3 | 5 — 8 | 70 ± 20 |
| 8 | 4000 | 600 | 4180 | 750 | 7 | 0.42 | 2 — 5 | 5 — 8 | 65 ± 15 |
| 9 | 3000 (a) | N/A | N/A | 800 | 57, 67 | 0.45 | 4 — 8 | 5 — 8 | 70 ± 20 |
| 10 | 4500 | 675 | 4770 | 700 | ¾” – No. 4 | 0.45 | 2 — 5 | 6 — 9 | 65 ± 15 |
| 11 | 4200 | 630 | 4420 | — | 57, 67 | 0.45 | 2 — 5 | 5 — 8 | 65 ± 15 |
| 12 | 4200 | 630 | 4420 | — | ¾” – No. 4 | 0.45 | 2 — 5 | 6 — 9 | 65 ± 15 |

Note 1: When concrete is exposed to water exceeding 15,000 ppm sodium chloride content, Type II cement shall be used.  In lieu ofType II cement, a Type I cement may be used in combined form with an amount of up to 50 percent replacement with ground iron blast furnace slag, or an amount of up to 25 percent replacement with Class F fly ash.  The Contractor shall submit to the Engineer the proposed mix proportions and satisfactory test results per C 1012 showing a sulfate resistance expansion not exceeding 0.10 percent at 180 days

Note 2: The temperature of Mix No. 6 when used for other than superstructure work as defined in TC-1.02 shall be 70 ± 20 F.

Note 3: Type A or D admixture shall be added to bridge, box culvert, and retaining wall concrete.

Note 4: Nonchloride Type C admixtures may be used when approved by the Engineer.

Note 5:	Other Slump Requirements:

When a high range water reducing admixture Type F or Type G is specified, the slump shall be 4 to 8 in.

When synthetic fibers are specified, the slump shall be 5 in. maximum.

When concrete is to be placed by the slip form method, the slump shall be 2-1/2 in. maximum.

When the absorption of the coarse aggregate is greater than 10 percent, the slump shall be 3 in. maximum.

Note 6: Mix 9 shall contain a Type F high range water reducing admixture.

Note 7: Mix 10 and 12 shall be proportioned as specified in 211.2 of the ACI’s Recommended Practices for Selection Proportions for. Structural Lightweight Concrete.  The maximum average Density of Cured Concrete shall be 118 lb/ft3. Control testing for Density of Cured Concrete shall be two companion cylinders for each 100 yd3, or fraction thereof, as specified in M 195.

Note 8: Mix 11 and 12 shall also conform to all requirements as specified in Table 902 C.

(a)		Acceptance will be based on a minimum compressive strength of 3000 psi in 24 hours. Design approval will be given   based on trial batch obtaining a minimum compressive strength of 2500 psi in 12 hours. Testing shall conform to 902.10.08 except that cylinders shall remain in the molds until tests are conducted.

Coarse and fine aggregate having an expansion up to 0.10 percent when tested for alkali silica reactivity (ASR) MSMT 212 may be used without restriction.  Aggregates having an expansion greater than 0.10 but less than 0.35 percent are considered reactive and may only be used when one of the options in table 902 B are employed. Those having an expansion of 0.35 percent and greater are prohibited.

60

TABLE 902 B

| OPTION | ALKALI CONTENT OF CEMENT % max | REPLACE CEMENT WITH |  | SPECIFICATION |
| --- | --- | --- | --- | --- |
|  |  | MATERIAL | % BY WEIGHT |  |
| 1 | 1.50 | Class F Fly Ash | 15 – 25 | M 295 |
| 2 | 1.50 | Ground Iron Blast Furnace Slag | 25 – 50 | M 302 Grade 100 or 120 |
| 3 | 1.50 | Microsilica | 5 – 7 | C 1240 |
| 4 | — | Blended Cement (a) | 100 | M 240 |
| 5 | 0.60 (b) | Low Alkali Cement | 100 | M 85 |

(a)	Pozzolan content of 15 – 25 percent by weight of cement

(b)	For mix 9 used for Portland cement concrete pavement repairs; the maximum allowable percentage of alkalies in Portland cement shall be 0.70.

When reactive aggregate is used, designate which option will be used to control the formation of the ASR gel.  If an option other than option 5 in Table 902 B above is chosen, conduct tests per MSMT 212 using the reactive aggregate and the proposed cementitious material.  The expansion test results shall not be greater than 0.10 percent.  When more than one reactive aggregate is used in a concrete mix, each shall be tested individually and the maximum amount of pozzolan required to reduce the expansion of all the aggregates to 0.10 percent or less shall be used.  Submit the aggregate source, test results, and the percent and type of replacement cement to the Engineer.  The Engineer may withhold source approval pending verification testing.

61

# TABLE 902 C

| MIX PHYSICAL PROPERTIES |  |  |
| --- | --- | --- |
| TEST PROPERTY | TEST METHOD | SPECIFICATION LIMITS |
| Minimum Cementitious Materials Factor, lb/yd3 | — | 580 |
| Maximum Content of Portland Cement, lb/yd3 | — | 550 |
| Water/Cementitious Materials Ratio by Wt. | — | 0.45 |
| Corrosion Inhibitor, gal/yd3 | 902.06.05 | 2.0 |
| Synthetic Fibers, lb/yd3 | 902.15 | 1.5 |
| Permeability of Field Concrete, moving average of three tests, coulombs max | T 277 Modified | 2500 |
| Permeability of Field Concrete, individual test, coulombs max | T 277 Modified | 3000 |
| Shrinkage at 28 days, microstrains | C 157 | 400 |

Note 1:  Only Type I or II Portland cement shall be used.

Note 2:  Mixes shall contain ground iron blast furnace slag, fly ash or microsilica.

Note 3: The water to cement ratio shall be based upon the total water to cementitious materials ratio.  The gallonage of the 	corrosion inhibitor shall be included in the water/cementitious materials ratio.

Note 4: The permeability test value of field concrete shall be the average of two test specimens representing production concrete. Test specimens shall be molded on the project site in 4 x 8 in. molds conforming to M 205.  Test specimens shall be handled under same conditions as compressive strength test specimens in conformance with C 31 for the first seven days. When seven days old, they shall be cured in a 100 F water bath for the remainder of the 28 day curing.  The 28 day rapid chloride permeability of the specimens will be determined in conformance with T 277. Test for the geometry of test specimens will be waived.

Note 5:  Shrinkage tests will be performed on trial mixes only.

Note 6:  High range water reducing admixture may be used except the water reducing requirements will be waived.

Note 7:  A sealer conforming to 902.12 shall be used on the finished surface.

902.10.04 Trial Batch.  A trial batch shall be prepared to certify that each mix meets 902.10.05 and 902.10.06.  Approval will be given when the test results meets the minimum required average strength.

Make arrangements with the AME at least two weeks in advance, to have an authorized representative present during the batching and testing.  Each trial batch shall consist of at least 3 yd3 of concrete.  Supply all equipment, and labor required to produce the trial batches and conduct the required tests at no additional cost to the Administration.  The AME may waive the requirement for a trial batch when past performance records show that the required average strength requirement has been met.

62

902.10.05 Design Required Average Strength.

| Specified compressive strength, fc´, psi | Required average compressive strength, fcr´, psi |
| --- | --- |
| fc´ ≤ 5000 | Use the larger value computed from Eq. (A-1) and (A-2) fcr´ = fc´ + 1.34s (A-1) fcr´ = fc´ + 2.33s – 500 (A-2) |
| Over 5000 | Use the larger value computed from Eq. (A-1) and (A-3) fcr´ = fc´ + 1.34s (A-1) fcr´ = 0.90 fc´ + 2.33s (A-3) |

where:

fc´ = the 28 day specified compressive strength.

s   = the standard deviation as specified in 902.10.06.

A test is defined as the average strength of two companion cylinders.

902.10.06 Standard Deviation.

(a)	When past performance records are available, a standard deviation will be established from documented performance records of the producer consisting of a minimum of 15 consecutive 28 day compressive strength tests obtained within the last 12 months.

The standard deviation will be established as the product of the calculated standard deviation and multiplier.

| # NUMBER OF TESTS | # MULTIPLIER FOR STANDARD # DEVIATION |
| --- | --- |
| 15 | 1.16 |
| 20 | 1.08 |
| 25 | 1.03 |
| 30 or more | 1.00 |
| Interpolate for intermediate number of tests. |  |

63

(b)	When past performance records are not available, the required average strength shall meet to the following:

| Specified compressive strength, fc´, psi | Required average compressive strength, fcr´, psi |
| --- | --- |
| fc´ < 3000 | fcr´ = fc´ + 1000 |
| 3000 ≤ fc´ ≤ 5000 | fcr´ = fc´ + 1200 |
| fc´ > 5000 | fcr´ = 1.10 fc´ + 700 |

902.10.07 Standard of Control.  The average of all sets of three consecutive strength tests shall equal or exceed the critical value as specified in 902.10.03 which shall be computed using the following formula:

Critical Value = fc' + (1.14 X S) – 500

Failure to conform to this criteria shall be cause for immediate investigation and remedial action up to and including suspension of production.  A design standard deviation equal to 15 percent of the specified strength shall be used for calculation until a minimum of 15 test results are obtained.

The actual average strength and standard deviation shall be computed upon the availability of 28 day strength data comprising a minimum of 15 tests.  Should this determination indicate an excessive margin of safety, the concrete mix may be modified to produce lower average strength as approved by the Engineer.  If these calculations indicate a coefficient of variation greater than 15, the quality of the concrete and testing will be evaluated.

64

902.10.08 Testing.  Sampling per T 141.  Testing as follows:

| TEST | METHOD | # MINIMUM TEST FREQUENCY | # RESPONSIBILITY |
| --- | --- | --- | --- |
| Temperature (e) | T 309 | # 1 per 50 yd3 (or fraction thereof) | # Project Engineer |
| Slump (a)(e) | T 119 | 1 per 50 yd3 (or fraction thereof) | Project Engineer |
| Air Content (a)(e) | T 152 T 196 | 1 per 50 yd3 (or fraction thereof) | Project Engineer |
| Compression (b)(c)(d) | T 23 | 1 per 50 yd3 (or fraction thereof) | Project Engineer |
| Compression (b)(c)(d) Mix No. 7 Only | T 23 | 3 per Day | Project Engineer |

(a)		A second test will be made when the first slump or air content test fails.  Acceptance or rejection will be based on the results of the second test.

(b)		Compressive strength tests are defined as the average of two companion cylinders.

(c)		The Contractor shall be responsible for the making of all early break cylinders and furnishing the molds, stripping, curing/delivery of all cylinders, including 28 day cylinders, to the testing laboratory.

(d)		The Project Engineer will be responsible for making, numbering and signing the 28 day cylinders.

(e)		When constructing plain and reinforced concrete pavements, the testing frequency for slump, air content, and temperature shall be 1 per 100 yd3 or fraction thereof.

902.10.09 Acceptance.  Concrete will be acceptable if both of the following requirements are met:

(a)	The average of all sets of three consecutive strength tests equal or exceed the specified design strength.

(b)	No individual strength test (average of two companion cylinders) falls below the specified design strength by more than 500 psi.

902.10.10 Price Adjustment.  A price adjustment will be based on the Contract unit price per cubic yard of concrete.  If the unit is a lump sum item, the price per cubic yard for the concrete will be determined by dividing the cubic yards into the Contract lump sum price.

(a)	Test Results More Than 500 psi Below the Specified Design Strength.  Failing strength tests will be considered individually with a price adjustment being applied on the percentage basis as shown below.

(Price per yd3) X (quantity of yd3 represented by the failing concrete strength) X (percent of failure).

Example: $400.00 per yd3 X 50 yd3 X [1-(3600/ 4500 psi)] = $4,000.00

65

No payment will be allowed when the test results fall below 50 percent of the specified design strength for structural concrete or 40 percent for incidental concrete.

The Engineer will determine when the strength of the concrete represented by the failing tests is sufficient to remain in place or whether it must be removed and replaced with Specification concrete.

- Test Results 500 psi or Less than the Specified Design Strength.  Strength failures 500 psi or less than the specified design strength will be averaged with the next two consecutive tests.  If those two tests include a failure greater than 500 psi, those tests will be evaluated as in 902.10.10(a) and replaced with the next consecutive test.  If the resulting average falls below the specified design strength, a price adjustment will be applied as specified in the table below.  Any failure will only be included in one grouping.

| STRENGTH BELOW THE SPECIFIED (avg of 3 tests) DESIGN LEVEL, psi | ADJUSTMENT FACTOR |
| --- | --- |
| MIX NO. 1 THRU MIX NO. 7 |  |
| 1 – 100 | 0.005 |
| 101 – 200 | 0.01 |
| 201 – 300 | 0.02 |
| 301 – 400 | 0.04 |
| 401 – 500 | 0.08 |

Adjustment price equals (price per yd3) X (quantity of yd3 represented by the failing cylinders) X (the adjustment factor).

Example:

$400.00 per yd3 X 50 yd3 X 0.01 = $200.00

902.11 MORTAR FOR GROUT.  Mortar used for grouting anchor bolts, pipe, handrail posts, and miscellaneous items shall be composed in accordance with one of the following:

(a)	One part Portland cement or blended hydraulic cement and one part mortar sand by dry loose volume.

(b)	Prepared bag mixes consisting of Portland cement or blended hydraulic cement and mortar sand.  The prepared mixes shall produce a mortar meeting the strength requirements specified in the Contract Documents.

- Use nonshrink grout when specified.  The grout shall have a minimum compressive strength of 5000 psi in seven days when tested as specified per T 106, except that the cube molds shall remain intact with a top firmly attached throughout the curing period.  The nonshrink grout shall have a minimum expansion of 0.0 percent after seven days when tested as specified per T 160.

66

(d)	Epoxy grout shall consist of sand and epoxy mixed by volume in per the manufacturer's recommendations.  The grout shall be capable of developing a minimum compressive strength of 6500 psi in 72 hours when tested per MSMT 501.  Sand for epoxy grout as specified in 901.01.

(e)	An epoxy or polyester anchoring system may be used when approved by the Engineer in accordance with the manufacturer's recommendations.  Strength values shall be as specified in the Contract Documents.

902.12 LINSEED OIL.  Shall consist of a 50-50 mixture (by volume) of boiled linseed oil meeting Federal Specification TT-L-190 and kerosene per D 3699.

902.13 LATEX MODIFIED CONCRETE.  Portland cement concrete containing prequalified Laboratory approved styrene butadiene latex emulsion is defined as Latex Modified Concrete (LMC).

Latex emulsion shall have a minimum of 90 percent of the nonvolatiles as styrene butadiene polymers.  The latex emulsion as specified in Table 902.13 A.  The material shall be stored in suitable containers and be protected from freezing and exposure to temperatures in excess of 85 F.

LMC shall be proportioned using volumetric mixing and designed as follows:

| # LATEX MODIFIED CONCRETE |  |
| --- | --- |
| MATERIAL | SPECIFICATION LIMITS |
| Portland Cement, CWT/yd3, min | 6.6 |
| Latex Emulsion/Cement Ratio | 0.31 – 0.34 |
| Water/Cement Ratio, max | 0.22 |
| Entrained Air, % | 6.0 3 |
| Slump, in. | 5 1 |

The physical properties of LMC shall conform to Table 902.13 B.  The Contractor shall furnish the necessary 3 X 6 in. molds per M 205 to be used for the fabrication of compressive strength cylinders.

Control and Acceptance Sampling.

(a)	Submit a two qt minimum sample, of the styrene butadiene latex emulsion to the AME daily for each lot of material used in a day's production.

(b)	A batch for LMC is defined as the capacity of the equipment being used on the project.  Slump and air samples will be taken and tested before the placement of a batch is permitted.  The slump shall be measured four to five minutes after discharge from the mixer.  The test material shall be deposited off the deck and not be disturbed during this

67

waiting period.  One additional sample for slump and air will be taken randomly during the placement of each batch.  For seven day compressive strength, two tests each per batch are required.  A test is defined as consisting of two companion cylinders.  The samples for these tests will be taken at random while the placement is in progress.

TABLE 902.13 A

| REQUIREMENTS FOR CHEMICAL PROPERTIES OF LATEX EMULSION MATERIALS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PROPERTY | SPECIFICATIONS |  | QUALITY ASSURANCE TESTS |  |
|  | LIMITS | TOLERANCE | PRE-QUALIFICATION TESTS | CONTROL AND ACCEPTANCE |
| Color | White | — | X | X |
| pH | 9.0 – 11.0 | — | X | X |
| Weight, lb/gal | 8.40 – 8.47 | — | X | X |
| Solids Content, % | 46 – 53 | — | X | X |
| *Butadiene Content, % of polymer | 30 – 40 | — | — | — |
| Viscosity @ 10 rpm-cps | Match Original | 20 | X | X |
| *Surface Tension, dynes/cm max | 50 | — | — | — |
| *Mean Particle Size, polymer – Å | 1400 – 2500 | — | — | — |
| Coagulum, % max | 0.10 | — | X | X |
| *Freeze-Thaw Stability, coagulum, % max | 0.10 | — | X | X |
| Infrared Spectra of Latex Film | Match Original | — | X | X |
| Infrared of Alcohol, Soluble Portion of Latex | Match Original | — | X | X |
| Shelf Life, min | 1 yr | — | X | — |

Note 1:	Quality assurance tests shall be conducted as specified in MSMT 612 except those denoted by an * shall be conducted as specified in FHWA RD – 78-35.

Note 2:	The original or prequalification sample shall be accompanied by the producer's certification on all of the tests and properties noted above and as specified in TC-1.02.  The certification shall contain actual test values of the product and the infrared spectrograph.

Note 3:	A separate certification is required for each lot of material.  The certification shall note the date of manufacture, lot size, and whether or not the material is identical to the formulation of the original sample.

68

### TABLE 902.13 B

| # LATEX MODIFIED CONCRETE # PHYSICAL PROPERTIES |  |  |  |
| --- | --- | --- | --- |
| # TEST PROPERTY | TEST VALUES | # QUALITY # ASSURANCE TESTS |  |
|  |  | PRE-QUALIFIED TESTS | CONTROL AND ACCEPTANCE |
| 7 Day Compressive Strength, psi min | 3000 | X | X |
| 28 Day Compressive Strength, psi min | 3500 | X | — |
| 42 Day Compressive Strength, psi min | 3500 | X | — |
| 7 Day Flexural Strength, psi min | 550 | X | — |
| 28 Day Flexural Strength, psi min | 650 | X | — |
| 42 Day Shear Bond Strength, psi min | 2000 | X | — |
| Durability Factor, 300 cycles, % min | 85 | X | — |
| Chloride Permeability, Ppm max | 510 | X | — |
| Scaling Resistance, 50 cycles, max | 3 | X | — |

Note 1:  Quality assurance tests shall be conducted as specified in MSMT 721.

Note 2: Seven Day Compressive Strength Test will be used for Control & Acceptance of the material.  The minimum   specified design strength is 3000 psi at seven days.  The mix design approval and acceptance will be based on a coefficient of variation of 10 percent with a probability of 1 in 10 tests falling below the specified strength.  Only  test values 80% or greater than the specified strength will be accepted

902.14 RAPID HARDENING CEMENTITIOUS MATERIALS FOR CONCRETE PAVEMENT REPAIRS.  Materials shall be a dry, packaged cementitious mortar having less than 5 percent by weight of aggregate retained on the 3/8 in. sieve and meet the following requirements:

Classification.

Class I — For use at ambient temperatures below 50 F.

Class II — For use at ambient temperatures of 50 to 90 F.

Class III — For use at ambient temperatures above 90 F.

Chemical Requirements.  C 928 except that no organic compounds such as epoxy resins or polyesters as the principal binder.

69

Physical Requirements.  Meet the following when tested per MSMT 725:

| #### COMPRESSIVE STRENGTH, psi min |  |  |  |  |
| --- | --- | --- | --- | --- |
| CLASSIFICATION | < 2 hr | 2-6 hr | 6 hr | 28 days |
| Type I — Slow | — | — | 2000 | 4500 |
| Type II — Rapid | — | 2000 | — | 4500 |
| Type III — Very Rapid | 2500 | — | — | 4500 |

| TEST RESULTS |  |
| --- | --- |
| TEST PROPERTY | LIMITS |
| Bond Strength, 7 days, psi min | 2000 |
| Length Change, increase after 28 days in water, based on length at 3 hr, % max | + 0.15 |
| Length Change, decrease after 28 days, % max | - 0.15 |
| Freeze Thaw, loss after 25 cycles in 10% CaCl2 solution, % max | 8 |
| Initial Setting Time, minutes min | 10 |

Marking.  All packages delivered to the project shall be marked with the following information:

(a)	Date material was packaged.

(b)	Approximate setting time.

(c)	Recommended dosage of water or liquid component.

(d)	Mixing instructions.

(e)	Class or temperature range.

Certification.  The manufacturer shall furnish certification as specified in TC-1.02 showing the actual test results for each class and type of material submitted to the Laboratory.

902.15 SYNTHETIC FIBERS.  When synthetic fibers are specified in the Contract Documents, the fibers shall be 1/2 to 1-1/2 in. long and conform to C 1116, Type III. The manufacturer shall furnish certification as specified in TC-1.02.  The quantity of fibers used and their point of introduction into the mix shall conform to the fiber manufacturer's recommendations.

70

CATEGORY 900

MATERIALS

740		DELETE:  SECTION 916 — SOIL AND SOIL-AGGREGATE BORROW in its entirety.

INSERT:  The following.

SECTION 916 — SOIL AND SOIL-AGGREGATE

BORROW

916.01 BORROW EXCAVATION.  A soil or soil aggregate mixture meeting the following:

Maximum dry density and optimum moisture content of the material per T 180, Method C unless the material has more than 35 percent retained on the No. 4 sieve, in which case Method D shall be used.  Material with a maximum dry density of less than 100 lb/ft3 is unsatisfactory and shall not be used in embankments.  Potentially expansive materials, such as steel slag, are prohibited.

Refer to the Recycled Materials Special Provisions located elsewhere in the Contract Documents.

| BORROW REQUIREMENTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Class Borrow | Max Dry Density Minimum P.C.F. T 180 | LL Maximum T 89 | PI Maximum T 90 | Gradation Requirements T 88 | Reference MSMT Soil Classification | Reference AASHTO Classification |
| Select Borrow | 105 | 34 | 7 | 30% max passing No. 200 sieve | A-2,A-3, A-2-4 | A-1-a, A-1-b, A-3, A-2-4 |
| Capping Borrow | 105 | 34 | 7 | 30% max passing No. 200 sieve* | A-2,A-3, A-2-4 | A-1-a, A-1-b, A-3, A-2-4 |
| Modified Borrow | 125 | 30 | 9 | 50% min.retained on No. 4 sieve | Any material except A-5 | A-2-4, A-4** |
| Common Borrow | 100 | N/A | N/A | N/A | N/A | N/A |
| * When material has no liquid and plastic limit, and the amount of material that passes the No 4 sieve and retained on the No. 10 sieve is less that 10 percent of the total sample mass, the material shall have at least 15 percent passing the No. 200 sieve. ** When A-4, the material has to be a manufactured product. |  |  |  |  |  |  |

CATEGORY 900

MATERIALS

789	DELETE:  SECTION 925 — DETECTABLE WARNING SURFACES in its entirety.

INSERT:  The following.

SECTION 925  DETECTABLE WARNING SURFACES

925.01 GENERAL.  Detectable warning surfaces shall conform to the current accessibility guidelines of the Americans with Disabilities Act (ADA).  The Office of Materials Technology (OMT) maintains a Qualified Products List (QPL).  Manufacturers seeking inclusion of their product on the QPL shall submit certified test results showing conformance to the properties in 925.07, as well as installation instructions and the types of adhesives and sealants required.

925.02 COMPOSITION.  Warning surfaces shall be either flexible or rigid.  If there is a change in the composition of a qualified product, the manufacturer shall notify OMT and submit new test results showing conformance with 925.07.

925.02.01 Pavers.  Type III Brick Pavers shall conform to the requirements of C 902, Class SX, Type 1, and Application PX.  The pavers shall be 2-1/4 x 4 x 8 in. with square edges and a surface meeting 925.03.

925.03 CONFIGURATION AND DIMENSIONS.  The warning surface shall consist of a system of truncated domes having a base diameter of 0.9 in. to 1.4 in., a top diameter 50 to 65 percent of the base diameter, and a height of 0.2 in.  The domes shall be arranged in a square grid with center-to-center spacing of 1.66 to 2.35 in.

925.04 COLOR.  The color shall be homogeneous across the surface of the material and contrast with adjoining surfaces.

925.05 IDENTIFICATION.  The top surface shall have an identifier that uniquely distinguishes the manufacturer.  Brick pavers are excluded.

925.06 REQUIREMENTS.

| TYPE | DESCRIPTION | PHYSICAL TEST REQUIREMENTS |
| --- | --- | --- |
| Type I | Cast in Place | A, B, C, D, E, G |
| Type IIa | Surface Mount, Rigid | A, B, C, D, E, G |
| Type IIb | Surface Mount, Flexible | A, B, C, D, F, G |
| Type III | Brick Pavers | 925.02.01 |
| Type IV | Prefilled Pavers | A, B, C, D, G |

925.07 PHYSICAL PROPERTIES.

|  | PROPERTY | TEST METHOD | SPECIFICATION LIMIT |
| --- | --- | --- | --- |
| A | Slip Resistance Coefficient | C 1028 (dry method) | 0.80 minimum |
| B | Abrasive Wear, index | C 501 | 150 minimum |
| C | Fade (UV) Resistance/Color Retention | D 4587 | Fade or Change in color after 2000 hours less than ΔE =5* |
| D | Freeze/Thaw Resistance | C 1026 | No disintegration |
| E | Adhesion/Bond Strength, pull off | C 482/C 882(as appropriate) | No adhesion failure |
| F | Adhesion/Bond Strength, peel | D 903/D 429 (modified as appropriate) | No adhesion failure |
| G | Contrast | Contrast percentage formula** using E 1349 to determine cap Y brightness/light reflectance values (LRV) | Current ADA requirement*** |

*	Chromaticity coordinates (L*a*b* system) checked in conformance with D 2244, before and after test.

**	Contrast % = [(B1 – B2)/B1] x 100,

where B1 = (LRV) of the lighter area, and B2 = (LRV) of the darker area.

***	For the purpose of determining whether a material meets acceptable contrast criteria, use actual cap Y brightness of detectable warning surface, and assume a value of 15 for the cap Y brightness of cured concrete, or a value of 3 for asphalt wearing surfaces to determine percentage difference.  Detectable warning surfaces to be installed on other materials are required to undergo additional testing.

THE CITY OF ABERDEEN

DEPARTMENT OF PLANNING AND COMMUNITY DEVELOPMENT

PROPOSAL FORM

Proposal by

Name

Address (Street and/or P.O. Box)

City	State	Zip

(	)	(	)

Phone No.		Fax No.

To furnish and deliver all materials and to do and perform all work, in conformance with the Standard Specifications, revisions thereto, General Provisions and the Special Provisions in this contract to the City of Aberdeen, located in Harford County, Maryland, for which Invitation for Bids will be received until 3:00 p.m. on November 18, 2013 this work being situated as follows:

City of Aberdeen 					Project Manager: Phyllis Grover

60 North Parke Street				pgrover@aberdeen-md.org

Aberdeen, MD 21001

410-272-1600 ext. 216

410-273-7402 (fax)

In response to the advertisement by the Administration, inviting bids for the work in conformance with the Contract Documents, now on file in the office of the Administration. I/We hereby certify that I/we am/are the only person, or persons, interested in this bid proposal as principals, and that an examination has been made of the work site, the Specifications, the Plans, and Invitation for Bids, including the Special Provisions contained herein.  I/We propose to furnish all necessary machinery, equipment, tools, labor and other means of construction, and to furnish all materials required to complete the project at the following unit price or lump sum price.

SCHEDULE OF PRICES

| "SCHEDULE OF PRICES" SHEETS MUST BE NUMBERED 2-1 OF , 2-2 OF , 2-3 OF , ETC. AND MUST BE INSERTED HERE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

GENERAL MATERIAL REQUIREMENTS

CONVICT PRODUCED MATERIALS

Section 1019 of the Intermodal Surface Transportation Efficiency Act of 1991 (ISTEA) clarifies that materials produced by convict labor after July 1, 1991 may not be used for Federal-aid highway construction projects unless produced at a prison facility producing convict made materials for Federal-aid construction projects prior to July 1, 1987.

CONTRACT PROVISION BUY AMERICA

The Contractor shall comply with Section 165 of the Surface Transportation Assistance Act of 1982 as amended by Section 1041(a) and 1048(a) of the Intermodal Surface Transportation Efficiency Act of 1991 with regard to the furnishing and coating of iron and steel products.  A nationwide waiver for this provision has been granted for pig iron and processed, pelletized, and reduced iron ore.

All bidders shall submit a bid using Domestic Iron and Steel Products with coatings that have been applied inside the United States.  If the bidder elects, an additional alternate bid may be submitted using Foreign Products on one or more of the above items in this Contract.

The Contract, if awarded, will be awarded to the responsible bidder who submits the lowest total bid for the Contract based on furnishing Domestic Products unless such bid exceeds the lowest total bid based on furnishing Foreign Products by more than twenty five percent (25%).  Foreign Products will not be permitted to be used as a substitution for Domestic ones after the bid has been awarded.

When steel and iron products and/or coatings are used in a project, the Contractor is not prohibited from using a minimal amount of foreign steel and iron products and/or coatings, if the cost of such materials used does not exceed one-tenth of one percent (0.1 %) of the total contract cost or $2,500, whichever is greater.

76

ALTERNATE BID

USING FOREIGN PRODUCTS

When a bidder elects to utilize Foreign Products on one or more items, the following summation indicating the Total Bid using Foreign Products must be completed in addition to the individual item bid tabulations.

The following instructions are given to the bidder in completing the Total Bid summation using Foreign Products:

1 -	The "Bid Total" for the initial bid using Domestic Products shall be shown on line (1).

2 -	The subtotal for Item Amounts using Domestic Products shall be shown on line (2), for those items which the Contractor elects to use Foreign Products.

3 -	The subtotal for Item Amounts using Foreign Products shall be shown on line (3).

4 -	The total Bid, utilizing Foreign Products shall be shown on line (4).  The value is obtained by subtracting subtotal (2) from the Total Bid (1) and then adding subtotal (3).

Bid Total for Bid 1 using	Line (1)

Domestic items

Total of Domestic Items	Line (2) -

Total of Foreign Items	Line (3) +

Bid Total using Foreign Items	Line (4)

77

5 of 28

ALTERNATE BID - USING FOREIGN PRODUCTS

BIDDER'S INSTRUCTIONS

When the bidder elects to submit a bid for one or more items using Foreign Products, the following form must be used.  For each item that Foreign Products are contemplated, the appropriate "Item Numbers", "Approximate Quantities", "Description of Items", "Unit Price or Lump Sum Price", "Item Amount Domestic" and "Item Amount Foreign" shall be tabulated below as specified in the initial bid.  The bidder shall indicate the unit price in dollars and cents and show the total cost of the item for each item that utilizes Foreign Products.  When all items utilizing Foreign Products have been listed, the bidder shall indicate on Page 4 of 20 the subtotals of the Item Amounts for Domestic Products in Line (2) and for Foreign Products in Line (3).

| Item Nos. | Approximate Quantities | Description of Items | Unit Price or Lump Sum Dollars & Cents |  | Items Amount Domestic Dollars & Cents |  | Items Amount Foreign Dollars.Cts. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

6 of 28

BID/PROPOSAL AFFIDAVIT

A.	AUTHORIZED REPRESENTATIVE AND AFFIANT

I HEREBY AFFIRM THAT:

I am the (title) _______________________________________ and the duly authorized representative of (business) ____________________________________ and that I possess the legal authority to make this Affidavit on behalf of myself and the business for which I am acting.

B. CERTIFICATION REGARDING COMMERCIAL NONDISCRIMINATION

The undersigned bidder or offeror hereby certifies and agrees that the following information is correct:

In preparing its bid on this project, the bidder or offeror has considered all proposals submitted from qualified, potential subcontractors and suppliers, and has not engaged in “discrimination” as defined in §19-103 of the State Finance and Procurement Article of the Annotated Code of Maryland.  “Discrimination” means any disadvantage, difference, distinction, or preference in the solicitation, selection, hiring, or commercial treatment of a vendor, subcontractor, or commercial customer on the basis of race, color, religion, ancestry, or national origin, sex, age, marital status, sexual orientation, or on the basis of disability or any otherwise unlawful use of characteristics regarding the vendors, supplier’s or commercial customer’s employees or owners.  “Discrimination” also includes retaliating against any person or other entity for reporting any incident of “discrimination”.  Without limiting any other provision of the solicitation on this project, it is understood that, if the certification is false, such false certification constitutes grounds for the State to reject the bid submitted by the bidder or offeror on this project, and terminate any contract awarded based on the bid.  As part of its bid or proposal, the bidder or offeror herewith submits a list of all instances within the past 4 years where there has been a final adjudicated determination in a legal or administrative proceeding in the state of Maryland that the bidder or offeror discriminated against subcontractors, vendors, suppliers, or commercial customers, and a description of the status or resolution of that determination, including any remedial action taken.  Bidder or Offeror agrees to comply in all respects with the State’s Commercial Nondiscrimination Policy as described under Title 19 of the State Finance and Procurement Article of the Annotated Code of Maryland.

79

C.  AFFIRMATION REGARDING BRIBERY CONVICTIONS

I FURTHER AFFIRM THAT:

Neither I, nor to the best of my knowledge, information, and belief, the above business (as is defined in Section 16-101(b) of the State Finance and Procurement Article of the Annotated Code of Maryland), or any of its officers, directors, partners, controlling stockholders, or any of its employees directly involved in the business’s contracting activities, including obtaining or performing contracts with public bodies, has been convicted of, or has had probation before judgment imposed pursuant to Criminal Procedure Article, §6-220, Annotated Code of Maryland, or has pleaded nolo contendre to a charge of, bribery, attempted bribery, or conspiracy to bribe in violation of Maryland law, or of the law of any other state or federal law, except as follows (indicate the reasons why the affirmation cannot be given and list any conviction, plea, or imposition of probation before judgment with the date, court, official or administrative body, the sentence or disposition, the name(s) of person(s) involved, and their current positions and responsibilities with the business):

________________________________________________________________________ ________________________________________________________________________

________________________________________________________________________.

D. AFFIRMATION REGARDING OTHER CONVICTIONS

I FURTHER AFFIRM THAT:

Neither I, nor to the best of my knowledge, information, and belief, the above business, or any of its officers, directors, partners, controlling stockholders, or any of its employees directly involved in the business's contracting activities including obtaining or performing contracts with public bodies, has:

1.	Been convicted under state or federal statute of:

(a) a criminal offense incident to obtaining, attempting to obtain, or performing a public or private contract; or

(b) fraud, embezzlement, theft, forgery, falsification or destruction of records, or receiving stolen property;

2.	Been convicted of any criminal violation of a state or federal antitrust statute;

3.	Been convicted under the provisions of Title 18 of the United States Code for violation of the Racketeer Influenced and Corrupt Organization Act, 18 U.S.C. §1961, et seq., or the Mail Fraud Act, 18 U.S.C. §1341, et seq., for acts in connection with the submission of bids or proposals for a public or private contract;

80

4.	Been convicted of a violation of the State Minority Business Enterprise Law, Section 14-308 of the State Finance and Procurement Article of the Annotated Code of Maryland;

5.	Been convicted of a violation of the Section 11-205.1 of the State Finance and Procurement Article of the Annotated Code of Maryland;

6.	Been convicted of conspiracy to commit any act or omission that would constitute grounds for conviction or liability under any law or statute described in subsection (1) through (5) above;

7.	Been found civilly liable under a state or federal antitrust statute for acts or omissions in connection with the submission of bids or proposals for a public or private contract;

8.	Been found in a final adjudicated decision to have violated the Commercial Nondiscrimination Policy under Title 19 of the State Finance and Procurement Article of the Annotated Code of Maryland with regard to a public or private contract; or

9.	Admitted in writing or under oath, during the course of an official investigation or other proceedings, acts or omissions that would constitute grounds for conviction or liability under any law or statute described in Section B-C and subsections (1) through (8) above, except as follows (indicate reasons why the affirmations cannot be given, and list any conviction, plea, or imposition of probation before judgment with the date, court, official or administrative body, the sentence or disposition, the name(s) of the person(s) involved and their current positions and responsibilities with the business, and the status of any debarment):

________________________________________________________________________ ________________________________________________________________________ ________________________________________________________________________.

E.	AFFIRMATION REGARDING DEBARMENT

I FURTHER AFFIRM THAT:

Neither I, nor to the best of my knowledge, information, and belief, the above business, or any of its officers, directors, partners, controlling stockholders, or any of its employees directly involved in the business’s contracting activities, including obtaining or performing contracts with public bodies, has ever been suspended or debarred (including being issued a limited denial of participation) by any public entity, except as follows (list each debarment or suspension providing the dates of the suspension or debarment, the name of the public entity and the status of the proceedings, the name(s) of the person(s) involved and their current positions and responsibilities with the business, the grounds of the debarment or suspension, and the details of each person's involvement in any activity that formed the grounds of the debarment or suspension): ___________________________________________________

F. AFFIRMATION REGARDING DEBARMENT OF RELATED ENTITIES

I FURTHER AFFIRM THAT:

1.	The business was not established and it does not operate in a manner designed to evade the application of or defeat the purpose of debarment pursuant to Sections 16-101, et seq., of the State Finance and Procurement Article of the Annotated Code of Maryland; and

2.	The business is not a successor, assignee, subsidiary, or affiliate of a suspended or debarred business, except as follows (you must indicate the reasons why the affirmations cannot be given without qualification):

_______________________________________________________________________

_______________________________________________________________________

_______________________________________________________________________.

G. SUB-CONTRACT AFFIRMATION

I FURTHER AFFIRM THAT:

Neither I, nor to the best of my knowledge, information, and belief, the above business, has knowingly entered into a contract with a public body under which a person debarred or suspended under Title 16 of the State Finance and Procurement Article of the Annotated Code of Maryland will provide, directly or indirectly, supplies, services, architectural services, construction related services, leases of real property, or construction.

H. AFFIRMATION REGARDING COLLUSION

I FURTHER AFFIRM THAT:

Neither I, nor to the best of my knowledge, information, and belief, the above business has:

1.	Agreed, conspired, connived, or colluded to produce a deceptive show of competition in the compilation of the accompanying bid or offer that is being submitted;

2.	In any manner, directly or indirectly, entered into any agreement of any kind to fix the bid price or price proposal of the bidder or Offeror or of any competitor, or otherwise taken any action in restraint of free competitive bidding in connection with the contract for which the accompanying bid or offer is submitted.

82

I. FINANCIAL DISCLOSURE AFFIRMATION

I FURTHER AFFIRM THAT:

I am aware of, and the above business will comply with, the provisions of Section 13-221 of the State Finance and Procurement Article of the Annotated Code of Maryland, which require that every business that enters into contracts, leases, or other agreements with the State of Maryland or its agencies during a calendar year under which the business is to receive in the aggregate $100,000 or more shall, within 30 days of the time when the aggregate value of the contracts, leases, or other agreements reaches $100,000, file with the Secretary of State of Maryland certain specified information to include disclosure of beneficial ownership of the business.

J. POLITICAL CONTRIBUTION DISCLOSURE AFFIRMATION

I FURTHER AFFIRM THAT:

I am aware of, and the above business will comply with, Election Law Article, §§14-101—14-108, Annotated Code of Maryland, which requires that every person that enters into contracts, leases, or other agreements with the State of Maryland, including its agencies or a political subdivision of the State, during a calendar year in which the person receives in the aggregate $100,000 or more shall file with the State Board of Elections a statement disclosing contributions in excess of $500 made during the reporting period to a candidate for elective office in any primary or general election.

K. DRUG AND ALCOHOL FREE WORKPLACE

(Applicable to all contracts unless the contract is for a law enforcement agency and the agency head or the agency head's designee has determined that application of COMAR 21.11.08 and this certification would be inappropriate in connection with the law enforcement agency's undercover operations.)

I CERTIFY THAT:

1.	Terms defined in COMAR 21.11.08 shall have the same meanings when used in this certification.

2.	By submission of its bid or offer, the business, if other than an individual, certifies and agrees that, with respect to its employees to be employed under a contract resulting from this solicitation, the business shall:

83

(a) Maintain a workplace free of drug and alcohol abuse during the term of the contract;

(b)Publish a statement notifying its employees that the unlawful manufacture, distribution, dispensing, possession, or use of drugs, and the abuse of drugs or alcohol is prohibited in the business' workplace and specifying the actions that will be taken against employees for violation of these prohibitions;

(c) Prohibit its employees from working under the influence of drugs or alcohol;

(d) Not hire or assign to work on the contract anyone whom the business knows, or in the exercise of due diligence should know, currently abuses drugs or alcohol and is not actively engaged in a bona fide drug or alcohol abuse assistance or rehabilitation program;

(e) Promptly inform the appropriate law enforcement agency of every drug-related crime that occurs in its workplace if the business has observed the violation or otherwise has reliable information that a violation has occurred;

(f) Establish drug and alcohol abuse awareness programs to inform its employees about:

(i) The dangers of drug and alcohol abuse in the workplace;

(ii) The business' policy of maintaining a drug and alcohol free workplace;

(iii) Any available drug and alcohol counseling, rehabilitation, and employee assistance programs; and

(iv) The penalties that may be imposed upon employees who abuse drugs and alcohol in the workplace;

(g) Provide all employees engaged in the performance of the contract with a copy of the statement required by §K(2)(b), above;

(h) Notify its employees in the statement required by §K(2)(b), above, that as a condition of continued employment on the contract, the employee shall:

(i) Abide by the terms of the statement; and

(ii) Notify the employer of any criminal drug or alcohol abuse conviction for an offense occurring in the workplace not later than 5 days after a conviction;

(i) Notify the procurement officer within 10 days after receiving notice under §K(2)(h)(ii), above, or otherwise receiving actual notice of a conviction;

(j) Within 30 days after receiving notice under §K(2)(h)(ii), above, or otherwise receiving actual notice of a conviction, impose either of the following sanctions or remedial measures on any employee who is convicted of a drug or alcohol abuse offense occurring in the workplace:

84

(i) Take appropriate personnel action against an employee, up to and including termination; or

(ii) Require an employee to satisfactorily participate in a bona fide drug or alcohol abuse assistance or rehabilitation program; and

(k) Make a good faith effort to maintain a drug and alcohol free workplace through implementation of §K(2)(a)—(j), above.

3.	If the business is an individual, the individual shall certify and agree as set forth in §K(4), below, that the individual shall not engage in the unlawful manufacture, distribution, dispensing, possession, or use of drugs or the abuse of drugs or alcohol in the performance of the contract.

4.	I acknowledge and agree that:

(a) The award of the contract is conditional upon compliance with COMAR 21.11.08 and this certification;

(b) The violation of the provisions of COMAR 21.11.08 or this certification shall be cause to suspend payments under, or terminate the contract for default under COMAR 21.07.01.11 or 21.07.03.15, as applicable; and

(c) The violation of the provisions of COMAR 21.11.08 or this certification in connection with the contract may, in the exercise of the discretion of the Board of Public Works, result in suspension and debarment of the business under COMAR 21.08.03.

85

L. CERTIFICATION OF CORPORATION REGISTRATION AND TAX PAYMENT

I FURTHER AFFIRM THAT:

1.	 The business named above is a (domestic ___ ) (foreign __ ) corporation registered in accordance with the Corporations and Associations Article, Annotated Code of Maryland, and that it is in good standing and has filed all of its annual reports, together with filing fees, with the Maryland State Department of Assessments and Taxation, and that the name and address of its resident agent filed with the State Department of Assessments and Taxation is (IF NOT APPLICABLE, SO STATE):

Name: 		__________________________

Address:	__________________________

__________________________

__________________________

2.	Except as validly contested, the business has paid, or has arranged for payment of, all taxes due the State of Maryland and has filed all required returns and reports with the Comptroller of the Treasury, the State Department of Assessments and Taxation, and the Department of Labor, Licensing, and Regulation, as applicable, and will have paid all withholding taxes due the State of Maryland prior to final settlement.

M. CONTINGENT FEES

I FURTHER AFFIRM THAT:

The business has not employed or retained any person, partnership, corporation, or other entity, other than a bona fide employee, bona fide agent, bona fide salesperson, or commercial selling agency working for the business, to solicit or secure the Contract, and that the business has not paid or agreed to pay any person, partnership, corporation, or other entity, other than a bona fide employee, bona fide agent, bona fide salesperson, or commercial selling agency, any fee or any other consideration contingent on the making of the Contract.

86

N. ACKNOWLEDGEMENT

I ACKNOWLEDGE THAT this Affidavit is to be furnished to the Procurement Officer and may be distributed to units of: (1) the State of Maryland; (2) counties or other subdivisions of the State of Maryland; (3) other states; and (4) the federal government. I further acknowledge that this Affidavit is subject to applicable laws of the United States and the State of Maryland, both criminal and civil, and that nothing in this Affidavit or any contract resulting from the submission of this bid or proposal shall be construed to supersede, amend, modify or waive, on behalf of the State of Maryland, or any unit of the State of Maryland having jurisdiction, the exercise of any statutory right or remedy conferred by the Constitution and the laws of Maryland with respect to any misrepresentation made or any violation of the obligations, terms and covenants undertaken by the above business with respect to (1) this Affidavit, (2) the contract, and (3) other Affidavits comprising part of the contract.

I DO SOLEMNLY DECLARE AND AFFIRM UNDER THE PENALTIES OF PERJURY THAT THE CONTENTS OF THIS AFFIDAVIT ARE TRUE AND CORRECT TO THE BEST OF MY KNOWLEDGE, INFORMATION, AND BELIEF.

Date: ____________			By: ______________________________

(Authorized Representative and Affiant)

87

COMPREHENSIVE SIGNATURE PAGE 1 OF 2

THE BIDDER IS HEREBY NOTIFIED THAT THIS DOCUMENT SHALL BE SIGNED IN INK IN ORDER FOR THE BID TO BE ACCEPTED.  BY SIGNING, THE BIDDER CERTIFIES THAT HE/SHE WILL COMPLY IN EVERY ASPECT WITH THESE SPECIFICATIONS.

FURTHER, I DO SOLEMNLY DECLARE AND AFFIRM UNDER THE PENALTIES OF PERJURY THAT THE CONTENTS OF THIS AFFIDAVIT (PARAGRAPHS A-N) ARE TRUE AND CORRECT TO THE BEST OF MY KNOWLEDGE, INFORMATION, AND BELIEF.

This bid form shall be filled out legibly in ink or typed.  The bid, if submitted by an individual, shall be signed by an individual; if submitted by a partnership, shall be signed by such member or members of the partnership as have authority to bind the partnership; if submitted by a corporation the same shall be signed by the President and attested by the Secretary or an Assistant Secretary.  If not signed by the President as aforesaid, there must be attached a copy of that portion of the By-Laws, or a copy of a Board resolution, duly certified by the Secretary, showing the authority of the person so signing on behalf of the corporation.  In lieu thereof, the corporation may file such evidence with the Administration, duly certified by the Secretary, together with a list of the names of those officers having authority to execute documents on behalf of the corporation, duly certified by the Secretary, which listing shall remain in full force and effect until such time as the Administration is advised in writing to the contrary.  In any case where a bid is signed by an Attorney in Fact the same must be accompanied by a copy of the appointing document, duly certified.

IF AN INDIVIDUAL:

NAME:

Street and/or P.O. Box

City	State	Zip Code	Fed ID or SSN

(SEAL)

Signature	Date

Print Signature

WITNESS:

Signature

Print Signature

88

COMPREHENSIVE SIGNATURE PAGE 2 OF 2

IF A PARTNERSHIP:

NAME OF PARTNERSHIP:

Street and/or P.O. Box

City	State	Zip Code	Fed ID or SSN

BY:	(SEAL)

Member Signature	Date

Print Signature

TITLE:		WITNESS:

Signature

Print Signature

IF A CORPORATION:

NAME OF CORPORATION:

Street and/or P.O. Box

City	State	Zip Code	Fed ID or SSN

STATE OF INCORPORATION:

BY:	(SEAL)

Signature	Date

Print Signature

TITLE:		WITNESS:

Secretary’s Signature

Print Signature

89

MDOT DBE FORM A - (MDOT-OP 016-2)

FEDERALLY-FUNDED CONTRACTS (BIDS ONLY)

CERTIFIED DBE UTILIZATION AND FAIR SOLICITATION AFFIDAVIT

Page 1 of 2

This affidavit must be included with the bid.  If the bidder fails to accurately complete and submit this affidavit as required, the bid shall be deemed not responsive.

In connection with the proposal submitted in response to Solicitation No. IFB_Contract No, I affirm the following:

- DBE Participation (PLEASE CHECK ONLY ONE)

I have met the overall certified Disadvantaged Business Enterprise (DBE) participation goal of       percent (     %).  I agree that the DBE firms listed in the DBE Participation Schedule - Part 2 of the MDOT DBE Form B (Federally-Funded Contracts – Bids Only) will be used to accomplish the DBE participation goal for this Contract for at least the dollar amounts set forth therein.

OR

I conclude that I am unable to achieve the DBE participation goal.  I hereby request a waiver of the overall goal. Within 10 business days of receiving notice that our firm is the apparent awardee or as requested by the Procurement Officer, I will submit a written waiver request and all required documentation in accordance with COMAR 21.11.03.11.   I agree that the DBE firms listed in the DBE Participation Schedule - Part 2 of the MDOT DBE Form B (Federally-Funded Contracts – Bids Only) will be used to accomplish the DBE participation goal for this Contract for at least the dollar amounts set forth therein.

- Additional DBE Documentation

I understand that if I am notified that I am the apparent awardee or as requested by the Procurement Officer, I must submit the following documentation within 10 business days of receiving such notice:

(a) Outreach Efforts Compliance Statement (MDOT DBE Form C - Federally-Funded Contracts – Bids Only);

(b) Subcontractor Project Participation Statement (MDOT DBE Form D - Federally-Funded Contracts – Bids Only);

(c) DBE Waiver Request documentation per COMAR 21.11.03.11 (if waiver was requested); and

(d) Any other documentation required by the Procurement Officer to ascertain bidder’s responsibility in connection with the certified DBE participation goal.

I acknowledge that if I fail to return each completed document (in 2 (a) through (d)) within the required time, the Procurement Officer may determine that I am not responsible and therefore not eligible for contract award.

90

MDOT DBE FORM A - (MDOT-OP 016-2)

FEDERALLY-FUNDED CONTRACTS (BIDS ONLY)

CERTIFIED DBE UTILIZATION AND FAIR SOLICITATION AFFIDAVIT

Page 2 of 2

- Information Provided to DBE firms

In the solicitation of subcontract quotations or offers, DBE firms were provided not less than the same information and amount of time to respond as were non-DBE firms.

I solemnly affirm under the penalties of perjury that the information in this affidavit is true to the best of my knowledge, information and belief.

_________________________			________________________

Company Name					Signature of Representative

_________________________			________________________

Address						Printed Name and Title

_________________________			________________________

City, State and Zip Code				Date

SUBMIT THIS AFFIDAVIT WITH BID/PROPOSAL

91

MDOT DBE FORM B – (MDOT-OP017-2)

FEDERALLY-FUNDED CONTRACTS (BIDS ONLY)

DBE PARTICIPATION SCHEDULE

PART 1 – INSTRUCTIONS FOR DBE PARTICIPATION SCHEDULE

parts 2 and 3 must be included with the bid.  If the bidder fails to accurately complete and submit part 2 WITH the bid as required, the bid shall be deemed not responsive.

Page 1 of 3

*** STOP ***

FORM INSTRUCTIONS

PLEASE READ BEFORE COMPLETING THIS FORM

- Please refer to the Maryland Department of Transportation (MDOT) DBE Directory at www.mdot.state.md.us to determine if a firm is certified for the appropriate North American Industry Classification System (“NAICS”) Code and the product/services description (specific product  that a firm is certified to provide or specific areas of work that a firm is certified to perform).   For more general information about NAICS, please visit www.naics.com.   Only those specific products and/or services for which a firm is certified in the MDOT Directory can be used for purposes of achieving the DBE participation goal.

- In order to be counted for purposes of achieving the DBE participation goal, the firm must be certified for that specific NAICS (“DBE” for Federally-funded projects designation after NAICS Code).  WARNING:  If the firm’s NAICS Code is in graduated status, such services/products will not be counted for purposes of achieving the DBE participation goal.  Graduated status is clearly identified in the MDOT Directory (such graduated codes are designated with the letter “G” after the appropriate NAICS Code).

- Examining the NAICS Code is the first step in determining whether a DBE firm is certified and eligible to receive DBE participation credit for the specific products/services to be supplied or performed under the contract.  The second step is to determine whether a firm’s Products/Services Description in the DBE Directory includes the products to be supplied and/or services to be performed that are used to achieve the DBE participation goal.

- If you have any questions as to whether a firm is certified to perform the specific services or provide specific products, please call MDOT’s Office of Minority Business Enterprise at 1-800-544-6056 or send an email to mbe@mdot.state.md.us.

- The Contractor’s subcontractors are considered second-tier subcontractors.  Third-tier contracting used to meet a DBE goal is to be considered the exception and not the rule.  The following two conditions must be met before MDOT, its Modal Administrations and the Maryland Transportation Authority may approve a third-tier contracting agreement:  (a) the bidder must request in writing approval of each third-tier contract arrangement, and (b) the request must contain specifics as to why a third-tier contracting arrangement should be approved.  These documents must be submitted with the bid in Part 2 of this DBE Participation Schedule.

92

MDOT DBE FORM B – (MDOT-OP017-2)

FEDERALLY-FUNDED CONTRACTS (BIDS ONLY)

DBE PARTICIPATION SCHEDULE

PART 1 – INSTRUCTIONS FOR DBE PARTICIPATION SCHEDULE

Page 2 of 3

- For each DBE firm that is being used as supplier/wholesaler/regular dealer/broker/manufacturer, please follow these instructions for calculating the dollar amount of the subcontract for purposes of achieving the DBE participation goal:

- Is the firm certified as a broker of the products/supplies?  If the answer is YES, please continue to Item C.  If the answer is NO, please continue to Item B.

B.	Is the firm certified as a supplier, wholesaler, regular dealer, or manufacturer of such products/supplies?  If the answer is YES, continue to Item D.  If the answer is NO, continue to Item C only if the DBE firm is certified to perform trucking/hauling services under NAICS Codes 484110, 484121, 484122, 484210, 484220 and 484230.  If the answer is NO and the firm is not certified under these NAICS Codes, then no DBE participation credit will be given for the supply of these products.

C.	For purposes of achieving the DBE participation goal, you may count only the amount of any reasonable fee that the DBE firm will receive for the provision of such products/supplies - not the total subcontract amount or the value (or a percentage thereof) of such products and/or supplies.  In Column 4 of the DBE Participation Schedule, please state the amount of any reasonable fee that the DBE firm will receive for the provision of such products/services in Line 4.1.

D.	Is the firm certified as a manufacturer (refer to the firm’s NAICS Code and specific description of products/services) of the products/supplies to be provided?  If the answer is NO, please continue to Item E.  If the answer is YES, for purposes of achieving the DBE participation goal, you may count the total amount of the subcontract.  In Column 4 of the DBE Participation Schedule, please state the total amount of the subcontract in Line 4.1.

- Is the firm certified as a supplier, wholesaler and/or regular dealer?  If the answer is YES (i) if the DBE firm is furnishing and installing the materials and is certified to perform these services, please include in Line 4.1 the total value of the subcontract amount (including full value of supplies); or (ii) if the firm is only being used as a supplier, wholesaler and/or regular dealer or is not certified to install the supplies/materials, for purposes of achieving the DBE participation goal, you may only count sixty percent (60%) of the value of the subcontract for these supplies/products (60% Rule).  In Line 4.2 of the DBE Participation Schedule, please state amount of the subcontract for these supplies/products only (not installation) and sixty percent (60%) of such value.

93

MDOT DBE FORM B – (MDOT-OP017-2)

FEDERALLY-FUNDED CONTRACTS (BIDS ONLY)

DBE PARTICIPATION SCHEDULE

PART 1 – INSTRUCTIONS FOR DBE PARTICIPATION SCHEDULE

Page 3 of 3

- Please note that for USDOT-funded projects, a DBE prime may count towards its DBE participation goal, work performed by its own forces.  Include information about the DBE prime in Part 2.

- WARNING:  The percentage of DBE participation, computed using the dollar amounts in Column 4 for all of the DBE firms listed in Part 2, MUST equal at least the DBE participation goal as set forth in MDOT DBE Form A – Federally-Funded Contracts (Bids Only) for this solicitation.  If a bidder is unable to achieve the DBE participation goal, then the bidder must request a waiver in Form A or the bid shall be deemed not responsive.  You may wish to use the Worksheet shown below to assist you in calculating the percentages and confirming that you have met the applicable DBE participation goal.

### WORKSHEET

Total DBE Firm Participation Amount $_______________________________________________ (Add amounts listed for all DBE Firms in Column 4 of DBE Participation Schedule) Divide by Total Contract Amount ÷________________________________________________ ##### Percent Overall DBE Participation =_____________________________________

94

MDOT DBE FORM B – (MDOT-OP017-2)

FEDERALLY-FUNDED CONTRACTS (BIDS ONLY)

DBE PARTICIPATION SCHEDULE

PART 2 – DBE PARTICIPATION SCHEDULE

Part 2 must be included with the bid.  If the bidder fails to accurately complete and submit part 2 with the bid as required, the bid shall be deemed not responsive.

Page __ of ___

| Prime Contractor | Project Description | Solicitation Number |
| --- | --- | --- |

LIST INFORMATION FOR EACH CERTIFIED MBE SUBCONTRACTOR USED TO ACHIEVE THE DBE PARTICIPATION GOAL.

| COLUMN 1 | COLUMN 2 | COLUMN 3 | COLUMN 4 Unless the bidder requested a waiver in MDOT DBE Form A – Federally Funded Contracts (Bids Only) for this solicitation, the cumulative DBE participation for all DBE firms listed herein must equal at least the DBE participation goal set forth in Form A. |
| --- | --- | --- | --- |
| NAME OF DBE PRIME OR SUBCONTRACTOR AND TIER | CERTIFICATION NO. AND CLASSIFICATION | NAICS CODE/S NAICS Code/s of the specific products to be supplied or services to be performed by the DBE firm | FOR PURPOSES OF ACHIEVING THE DBE PARTICIPATION GOAL. State the dollar amount of the products/services in Line 4.1 except for those services or products where the DBE firm is being used as a wholesaler, supplier or regular dealer. For those items of work where the DBE firm is being used as a supplier, wholesaler and/or regular dealer complete Line 4.2 using the 60% Rule. |
| Please check if DBE firm is a third-tier contractor (if applicable). Please submit written documents in accordance with Section 5 of Part 1 - Instructions | Certification Number: Women-Owned African American-Owned Other DBE Classification |  | 4.1 TOTAL AMOUNT TO BE PAID TO THE SUBCONTRACTOR (EXCLUDING PRODUCTS/SERVICES FROM SUPPLIERS, WHOLESALERS, AND REGULAR DEALERS). $ 4.2 TOTAL AMOUNT TO BE PAID TO THE SUBCONTRACTOR FOR ITEMS OF WORK WHERE THE DBE FIRM IS BEING USED AS A SUPPLIER, WHOLESALER AND/OR REGULAR DEALER. (PLEASE REFER TO SECTION 6(E) IN PART 1 - INSTRUCTIONS). Total value of Supplies/Products $ X 60% (60% Rule) = $ (Amount for purposes of achieving the DBE Participation Goal). |

Please check if Continuation Sheets are attached.

95

MDOT DBE FORM B – (MDOT-OP017-2)

FEDERALLY-FUNDED CONTRACTS (BIDS ONLY)

DBE PARTICIPATION SCHEDULE

CONTINUATION SHEET

Page __ of ___

| Prime Contractor | Project Description | Solicitation Number |
| --- | --- | --- |

LIST INFORMATION FOR EACH CERTIFIED MBE SUBCONTRACTOR USED TO ACHIEVE THE DBE PARTICIPATION GOAL.

| COLUMN 1 | COLUMN 2 | COLUMN 3 | COLUMN 4 Unless the bidder requested a waiver in MDOT DBE Form A – Federally Funded Contracts (Bids Only) for this solicitation, the cumulative DBE participation for all DBE firms listed herein must equal at least the DBE participation goal set forth in Form A. |
| --- | --- | --- | --- |
| NAME OF DBE PRIME OR SUBCONTRACTOR AND TIER | CERTIFICATION NO. AND CLASSIFICATION | NAICS CODE/S NAICS Code/s of the specific products to be supplied or services to be performed by the DBE firm | FOR PURPOSES OF ACHIEVING THE DBE PARTICIPATION GOAL. State the dollar amount of the products/services in Line 4.1 except for those services or products where the DBE firm is being used as a wholesaler, supplier or regular dealer. For those items of work where the DBE firm is being used as a supplier, wholesaler and/or regular dealer complete Line 4.2 using the 60% Rule. |
| Please check if DBE firm is a third-tier contractor (if applicable). Please submit written documents in accordance with Section 5 of Part 1 - Instructions | Certification Number: Women-Owned African American-Owned Other DBE Classification |  | 4.1 TOTAL AMOUNT TO BE PAID TO THE SUBCONTRACTOR (EXCLUDING PRODUCTS/SERVICES FROM SUPPLIERS, WHOLESALERS OR REGULAR DEALERS). $ 4.2 TOTAL AMOUNT TO BE PAID TO THE SUBCONTRACTOR FOR ITEMS OF WORK WHERE THE DBE FIRM IS BEING USED AS A SUPPLIER, WHOLESALER AND/OR REGULAR DEALER. (PLEASE REFER TO SECTION 6(E) IN PART 1 - INSTRUCTIONS). Total value of Supplies/Products $ X 60% (60% Rule) = $ (Amount for purposes of achieving the DBE Participation Goal). |

Please check if Continuation Sheets are attached.

96

MDOT DBE FORM B – (MDOT-OP017-2)

FEDERALLY-FUNDED CONTRACTS (BIDS ONLY)

DBE PARTICIPATION SCHEDULE

part 3 – certification for Dbe participation schedule

parts 2 and 3 must be included with the bid, as directed in the solicitation.

I hereby affirm that I have reviewed the Products and Services Description (specific product that a firm is certified to provide or areas of work that a firm is certified to perform) set forth in the MDOT DBE Directory for each of the DBE firms listed in Part 2 of this DBE Form B for purposes of achieving the DBE participation goal that was identified in the DBE Form A that I submitted with this solicitation, and that the DBE firms listed are only performing those products/services/areas of work for which they are certified.  I also hereby affirm that I have read and understand the form instructions set forth in Part 1 of this DBE Form B.

I solemnly affirm under the penalties of perjury that the contents of Parts 2 and 3 of MDOT DBE Form B are true to the best of my knowledge, information and belief.

____________________________________	______________________________

Company Name						Signature of Representative

____________________________________		______________________________

Address						Printed Name and Title

____________________________________		______________________________

City, State and Zip Code				Date

97

INFORMATION REQUIRED TO BE SUBMITTED FOR FEDERALLY ASSISTED CONTRACTS:

(a)	Each bidder shall provide the following information:

NAME OF FIRM:

Street and/or P.O. Box

City	State	Zip Code

DBE		 Non-DBE	Age of the firm 	 years

Annual gross receipts per last calendar year 	<$500,000 	$500,000-1,000,000

$1,000,000-3,000,000 	$3,000,000-5,000,000 	$5,000,000-10,000,000

>$10,000,000

(b)	Each bidder shall provide the following information for each firm quoting or considered as subcontractors and/or suppliers:

NAME OF FIRM:

Street and/or P.O. Box

City	State	Zip Code

DBE		 Non-DBE	Age of the firm 	 years

Annual gross receipts per last calendar year 	<$500,000 	$500,000-1,000,000

$1,000,000-3,000,000 	$3,000,000-5,000,000 	$5,000,000-10,000,000

> $10,000,000

NAME OF FIRM:

Street and/or P.O. Box

City	State	Zip Code

DBE		 Non-DBE	Age of the firm 	 years

Annual gross receipts per last calendar year 	<$500,000 	$500,000-1,000,000

$1,000,000-3,000,000 	$3,000,000-5,000,000 	$5,000,000-10,000,000

> $10,000,000

98

NAME OF FIRM:

Street and/or P.O. Box

City	State	Zip Code

DBE		 Non-DBE	Age of the firm 	 years

Annual gross receipts per last calendar year 	<$500,000 	$500,000-1,000,000

$1,000,000-3,000,000 	$3,000,000-5,000,000 	$5,000,000-10,000,000

> $10,000,000

NAME OF FIRM:

Street and/or P.O. Box

City	State	Zip Code

DBE		 Non-DBE	Age of the firm 	 years

Annual gross receipts per last calendar year 	<$500,000 	$500,000-1,000,000

$1,000,000-3,000,000 	$3,000,000-5,000,000 	$5,000,000-10,000,000

> $10,000,000

NAME OF FIRM:

Street and/or P.O. Box

City	State	Zip Code

DBE		 Non-DBE	Age of the firm 	 years

Annual gross receipts per last calendar year 	<$500,000 	$500,000-1,000,000

$1,000,000-3,000,000 	$3,000,000-5,000,000 	$5,000,000-10,000,000

> $10,000,000

Submit additional copies of this page as page 26A of 28, 26B of 28, etc. as necessary, and place them as the last pages in the Invitation for Bids.  Place an “X” for “NO” on the last copy.  Any additional Copies:  	 NO  	 YES

99

EXTRA WORK, CONTRACT TIME, BONDING, LIQUIDATED DAMAGES, AND PROPOSAL GUARANTY

EXTRA WORK.  It is further proposed to do all "Extra Work" which may be required to complete the work contemplated at unit prices or lump sum prices to be agreed upon in writing prior to starting such extra work, or if such prices or sums cannot be agreed upon, to perform such work on a Force Account basis as specified in TC-7.03.

CONTRACT TIME.  To commence work as specified in the "Notice to Proceed" and to prosecute the work to complete the contract within/or before 120 calendar days.

Any delay in awarding or the execution of this contract will not be considered as a basis for any monetary claim, however, an extension of time may be considered by the Project Manager, if warranted.

LIQUIDATED DAMAGES.  The Contractor is hereby advised that liquidated damages in the amount of One hundred dollars ($100.00) per calendar day will be assessed for unauthorized extensions beyond the contracted time of completion.

100

|  | CITY OF ABERDEEN Department of Planning and Community Development 60 N. Parke Street Aberdeen, MD 21001 410.272.1600 |
| --- | --- |

CONTRACT AGREEMENT

This Agreement, made and entered into this      day of                  , 2013, by and between the City of Aberdeen, a body corporate and politic, existing under and by virtue of the laws of the City of Aberdeen, hereinafter referred to as the Owner, party of the first part,

AND

a Corporation, Partnership, Individual, doing business under the Laws of the State of Maryland, with its principal office at                                          hereinafter called the Contractor, party of the second part; and

The Owner, pursuant to the authority vested in it by the City Charter, did, by solicitation heretofore made in accordance with the provisions of said Acts, invite proposals for the:

SIDEWALK/ HANDICAP RAMPS, DRIVEWAY APRONS, AND CURB REPAIRS ON

MOUNT ROYAL AVENUE IN THE CITY OF ABERDEEN

BID NO.  13-02

CONTRACT NO. AX352B52

AND

in accordance with said solicitation and with the documents prepared by the City of Aberdeen and provided to the bidders, the Contractor submitted to the Owner a bid for the construction of the said work, and a contract was duly awarded by the Owner to the Contractor for the construction of the work for the price specified in the Contractor’s Bid Form, Bid No. 13-02,  and the same was duly approved by the Owner; and the Request For Bid, Specifications,	       bid, bid tabulation, and all other Contract Documents are made a pertinent part of this contract as if incorporated herein.

101

The Contractor agrees to provide and furnish all labor; all necessary tools, parts, machinery, and equipment; all utility and transportation services; and all materials, both expendable and permanent; and, at its own risk and expense, to construct and complete all items of work within the time specified, time being of the essence of this contract, all in accordance with the Request for Bid and Specifications as approved by the Owner, all other Contract Documents, and such instructions as may be issued by the Owner during the progress of the work, and to complete all items of work therein required to be done within the time specified.

The Contractor will pay and the Owner will accept, in full consideration for the performance of the Contractor's obligation hereunder the estimated amount of $                    .  The Contractor shall make payment request in writing at the completion of work and acceptance.

The Contractor, by executing this Agreement, declares and asserts that it has read each and every clause in each of the Contract Documents, which are hereby made a part hereof with like force and effect as though recited herein at length, and fully understands the meaning of same, and that, in connection therewith, he has examined the site of the work and fully understands the character of the work to be done under this Agreement and agrees to complete the work in the number of days set forth in the Contract Documents.

The Contractor agrees, simultaneously with the execution of this Agreement, to deliver to the Owner a Performance Surety in the full amount of the contract price; said corporate surety shall be satisfactory to the Owner; an original Certificate of Insurance satisfactory to the Owner certifying that adequate and complete insurance is carried by the Contractor, in complete accordance with the requirements of the Contract Documents.

The Contractor further agrees that there shall be no discrimination against any employee, any application for such employment, or any other person in carrying out this project because of race, religion, color, national origin, sex or age.

IN WITNESS WHEREOF, the Owner, by virtue of a motion duly passed by its governing body, has caused this Agreement to be signed and executed in the name of the Owner and for the

102

Owner and the Contractor and has caused this Agreement to be executed the day and year first above written.

ATTEST:						By:

Company Name

Date			Signature

Title

City of Aberdeen

Recommended for Approval

Phyllis Grover, Director Planning and

Community Development

Reviewed and Approved

Douglas Miller, City Manager

Approved

Michael E. Bennett, Mayor

103

|  | CITY OF ABERDEEN Department of Planning and Community Development 60 N. Parke Street Aberdeen, MD 21001 410-272-1600 |
| --- | --- |

BID FORM

Date: ___________________

This Bid of ____________________________________________ (hereinafter called "Bidder"), a corporation, organized and existing under the laws of the State of ________________________, a partnership or an individual doing business as _______________________________________ submitted to:

City of Aberdeen

Department of Planning and Community Development

60 N. Parke Street

Aberdeen, Maryland 21001

Attn: Phyllis Grover, Director of Planning and Community Development

To Whom It May Concern:

The Bidder in compliance with your Request for Proposal for sidewalk/ handicap ramps and curbs repairs on Mount Royal Avenue in the City of Aberdeen, Bid No.  13-02, has examined the Request for Proposal, related documents and the site of the proposed work, being familiar with all of the conditions surrounding the proposed project including the materials, supplies, and proposes to complete the project in accordance with the Contract Documents within the time set forth therein, and at the price stated below.  The price is to cover all expenses incurred in performing the work required under the Contract Documents of which this Bid is a part.

The Bidder declares that the attached Request for Proposal therein referred to has been carefully examined and is understood.  It is proposed and agreed, if the bid is accepted, to contract with the City of Aberdeen (Owner) to do the required work in the manner set forth in the Request for Proposal.

Bidder hereby agrees to commence work under this contact on or before a date to be specified in a written "Notice to Proceed" by the Owner and to fully complete within 120 days.

Bidder agrees to perform all the work described in the Request for Proposal and for the fixed price as set forth in this Bid Form.

The fixed price shall include all labor, materials, equipment, cleaning, shoring, removal, overhead, profit, insurance, etc., to cover the finished work.

Bidder understands that the Owner reserves the right to reject any or all Bids or part thereof.

The Bidder agrees that this Bid shall be good and may not be withdrawn for a period of 60 calendar days after the scheduled closing time for receiving Bids.

104

Upon receipt of written notice of the acceptance of this Bid, Bidder will execute a formal contract within 15 days and delivery of Performance Surety as required by the Request for Proposal.

The undersigned swears (or affirms) under the penalty of perjury that the Bidder, its agents, servants, and/or employees have not in any way colluded with anyone for and on behalf of the Bidder, or themselves, to obtain information that would give the Bidder an unfair advantage over others, nor have they colluded with anyone for and on behalf of the Bidder, or themselves, to gain favoritism in the award of any contract resulting from this Bid.

SIDEWALK/ HANDICAP RAMPS, DRIVEWAY APRONS, AND CURB REPAIRS ON MOUNT ROYAL AVENUE IN THE CITY OF ABERDEEN

BID NO.  13-02

PROPOSAL

Unit Price Per Square Foot for Concrete Sidewalk	$

Unit Price Per Linear Feet for Curb and Gutter		$

Unit Price Per Square Foot for Handicap Ramp

Detectable Warning Surface		$

Add/Alternative – Unit Price to move meters		$_______________________________

Total Contract Amount Bid 						$ ____________________________

The foregoing fixed price includes and covers the furnishing of all materials, labor, necessary machinery, tools, apparatus, and means for performing the work, and doing of all the above mentioned work as set forth, described and shown in the Request for Proposal, and other Contract Documents within the prescribed number of consecutive calendar days after service of written notice from the Owner to proceed with the work.

____________________________________________________________________________

Contractor's Name

____________________________________________________________________________

Signature										Title	Date

Address	_________________________________________________________

Telephone No. 	(_____) _____________________________________________

Maryland Contractor’s License Number ___________________________________

** END OF BID FORM **

105

CONCRETE MOUNTABLE CURB AND GUTTER

PLATE S-2

A BITUMINOUS SEAL SHALL BE PLACES AT ALL CURB JOINTS ALONG THE GUTTER PAN

106

7” COMBINATION CURB AND GUTTER

PLATE S-1

Area= 0.16  ± S.Y.    18.75 ± L.F.ic.y

A BITUMINOUS SEAL SHALL BE PLACED AT ALL CURBS JOINTS ALONG THE GUTTER PAN.

107

SIDEWALK RAMP DETAIL PERPENDICULAR

PLATE S-7

SIDEWALK RAMPS PARALLEL

PLATE S-8

SIDEWALK RAMPS PARALLEL

PLATE S-9

DETECTABLE WARNING SURFACE

PLATE S-10

STANDARD 5” SIDEWALK

CURB & GUTTER TRANSITION AT INLETS

PLATE S-36